For Cisco Internal Use only


1. Introducing the MC20x20V-20D


11.
Introducing the MC20x20V-20D


41.7.
Using the MC20x20V-20D


61.8.
Sharing resources – all resources can be used


92.
Software features


92.7.
Dynamic Bandwidth Sharing


102.8.
VDOC


132.9.
Powerboost


132.10.
Rate adapt


142.11.
Subscriber Traffic Management


153.
Configuration


163.7.
References


163.8.
Concepts


194.
Examples


204.7.
A basic MC20x20V configuration.


244.8.
A basic MC88V configuration.


284.9.
Shared (spanned) 8 channel bonding into 4 service areas


334.10.
Shared (spanned) 4 channel IPTV into 4 service areas


334.10.1.
Basic Multicast Forwarding


344.10.2.
Multicast DSID Forwarding


354.10.3.
Explicit Tracking


354.10.4.
Multicast QoS


374.10.5.
Multicast BPI+


384.10.6.
Multicast Authorization


384.10.7.
Static Multicast Session


394.10.8.
VIDEO server:


394.10.9.
UBR10012 configuration sample 1 (VDOC)


504.10.10.
UBR10012 configuration sample 2




The MC20x20V-20D is a new generation line card for the uBR10012 CMTS that provides full DOCSIS 3.0 capability to the uBR10012 CMTS.

[image: image1.emf]
· 20 Downstream Channels

· 5 Physical connectors, 4 channels per port

· 20 Integrated-Cable interfaces 

· 30 DS bonding groups

· 20 Upstream Channels

· Virtual interface and Frequency stacking supported

· Supports DOCSIS 3.0 Features

· DS bonding, US bonding, BPI etc. 

· 5 Mac Domains per line card with flexibleNote allocation of downstream and upstream to a MAC domain.
1.7. Using the MC20x20V-20D
The MC20x20V-20D can be used to provide 20 downstreams and 20 upstreams into a service area in a variety of configurations.

A basic configuration provides 4 channel upstream and 4 channel downstream bonding into 5 service areas supported by 4 upstream per service area. Up to EightNote active line cards can be installed in a uBR10012 chassis in non high availability mode thus up to 40 service areas can be supported per chassis: 160 downstream and 160 upstream configured as 4x4 channel bonding or as 4x4 for DOCSIS 2.0 service. 

[image: image2.wmf]MC

20

x

20

V

-

20

D

LINE 

CARD

us

us

us

us

SERVICE 

AREA 

1

usf

1

usf

0

dsf

1

dsf

2

dsf

3

dsf

4

usf

2

usf

3

PRIMARY

PRIMARY

PRIMARY

PRIMARY

 MAC 

Domain

4 

CHANNEL BONDING

PRIMARY

DOCSIS 

2

.

0 

SUPPORT

us

us

us

us

SERVICE 

AREA 

2

PRIMARY

PRIMARY

PRIMARY

PRIMARY

 MAC 

Domain

us

us

us

us

SERVICE 

AREA 

3

PRIMARY

PRIMARY

PRIMARY

PRIMARY

 MAC 

Domain

us

us

us

us

SERVICE 

AREA 

4

PRIMARY

PRIMARY

PRIMARY

PRIMARY

 MAC 

Domain

us

us

us

us

SERVICE 

AREA 

5

PRIMARY

PRIMARY

PRIMARY

PRIMARY

 MAC 

Domain


But this is not the only variation that can be supported.

The MC20x20V-20D can support up to 8 upstream per MAC domain just as the older MC5x20H-D line card. In addition two upstream channels can be frequency stacked on the one connector but note that there is still a limit of 20 upstreams per line card thus only 10 physical connectors can be frequency stacked.

As can be seen from the following configuration, the MC20x20V-20D can be configured to provide 8 channel downstream bonding. 

[image: image3.wmf]MC

20

x

20

V

-

20

D

LINE 

CARD

us

us

us

us

SERVICE 

AREA 

1

usf

1

usf

0

dsf

1

dsf

2

dsf

3

dsf

4

usf

2

usf

3

PRIMARY

PRIMARY

PRIMARY

PRIMARY

 MAC 

Domain

4 

CHANNEL BONDING

PRIMARY

DOCSIS 

2

.

0 

SUPPORT

us

us

us

us

PRIMARY

PRIMARY

PRIMARY

PRIMARY

us

us

us

us

SERVICE 

AREA 

2

 MAC 

Domain

us

us

us

us

SERVICE 

AREA 

4

 MAC 

Domain

us

us

us

us

PRIMARY

PRIMARY

PRIMARY

PRIMARY

 MAC 

Domain

dsf

5

dsf

6

dsf

7

dsf

8

8 

CHANNEL BONDING

PRIMARY

PRIMARY

PRIMARY

PRIMARY

PRIMARY

PRIMARY

PRIMARY

PRIMARY


The line card will support 2 service areas with 8x8 and one service area of 4x4. If only 8 channel bonding is required then a 4x4 service area is effectively stranded (not used) and this is often not desirable.

In fact up to 20 channel downstream bonding can be configured by configuring all downstream to be available in the one 8 upstream service area but again we will be stranding 12 upstream connectors and again this is not desirable.

1.8. Sharing resources – all resources can be used

The MC20x20V-20D line card can be configured in a sharing mode of operation to make best use of all the upstream and downstream resources (no stranded resources) thus providing the best cost benefit to a customer. 
The level of sharing is only limited by the following rules:

· Sharing within one line card only

· A downstream can be primary in one MAC domain only

· The number of available downstream channels in the HFC plant downstream spectrum

In the following example, downstream resources are shared to provide 8 channel bonding in each of the 5 service areas supported by a single line card. 


[image: image4.wmf]Frequency

MC

20

x

20

V

-

20

D

LINE 

CARD

us

us

us

us

 MAC 

Domain

SERVICE 

AREA 

1

usf

1

usf

0

dsf

1

dsf

2

dsf

3

dsf

4

usf

2

usf

3

us

us

us

us

us

us

us

us

 MAC 

Domain

us

us

us

us

 MAC 

Domain

 MAC 

Domain

 MAC 

Domain

SHARED 

8 

CHANNEL BONDING

PRIMARY

DOCSIS 

2

.

0 

SUPPORT

SERVICE 

AREA 

2

SERVICE 

AREA 

3

SERVICE 

AREA 

4

us

us

us

us

SERVICE 

AREA 

5

dsf

5

dsf

6

dsf

7

dsf

8

PRIMARY

PRIMARY

PRIMARY

PRIMARY

PRIMARY

 
The following configuration is another variation on shared downstreams. This configuration uses all available upstream and downstream resources in each line card and provides:

5 upstream per service area and thus up to 5 channelNote upstream bonding in 4 service areas;

· 4 channel downstream bonding into 4 service areas 
· 4 DOCSIS 2.0 capable downstream into 4 service areas 
· DOCSIS 2.0 and DOCSIS 3.0 capacity shared across the 4 primary downstreams
· 8 channel bonding in each of the 4 service areas with capacity shared across the 4 service areas and;
· Shared 4 channel broadcast (IPTV) into each of the 4 service areas with broadcast capacity shared with the 8 channel bonding.
This configuration can be used to support DOCSIS 2.0 modems, 8x4 DOCSIS 3.0 modems and linear IPTV into the 4 service areas.

[image: image5.wmf]Frequency

MC

20

x

20

V

-

20

D

LINE 

CARD

us

us

us

us

 MAC 

Domain

SERVICE 

AREA 

1

usf

1

usf

0

dsf

4

dsf

5

dsf

6

dsf

7

usf

2

usf

3

us

us

us

us

PRIMARY

PRIMARY

PRIMARY

PRIMARY

us

us

us

us

 MAC 

Domain

us

us

us

us

 MAC 

Domain

PRIMARY

PRIMARY

PRIMARY

PRIMARY

PRIMARY

PRIMARY

PRIMARY

PRIMARY

PRIMARY

PRIMARY

PRIMARY

PRIMARY

 MAC 

Domain

 MAC 

Domain

4 

CHANNEL BONDING

SHARED 

8 

CHANNEL BONDING

SHARED 

4 

CHANNEL BONDED IPTV

us

PRIMARY

DOCSIS 

2

.

0 

SUPPORT

us

SERVICE 

AREA 

2

us

us

SECONDARY

SECONDARY

SECONDARY

SECONDARY

SERVICE 

AREA 

3

SERVICE 

AREA 

4

dsf

1

dsf

2

dsf

3

dsf

4


There are many such variations possible. Another option is shown following: 
· 3 upstream per service area of 8,6 and 6 upstreams

· 8 channel downstream bonding into all service areas 

· Minimum of 4 DOCSIS 2.0 capable downstream into all service areas 

· DOCSIS 2.0 and DOCSIS 3.0 capacity shared across the 4 primary downstreams as required


[image: image6.wmf]MC

20

x

20

V

-

20

D

LINE 

CARD

us

us

us

us

SERVICE 

AREA 

1

usf

1

usf

0

dsf

1

dsf

2

dsf

3

dsf

4

usf

2

usf

3

PRIMARY

PRIMARY

PRIMARY

PRIMARY

 MAC 

Domain

4 

CHANNEL BONDING

PRIMARY

DOCSIS 

2

.

0 

SUPPORT

us

us

us

us

PRIMARY

PRIMARY

PRIMARY

PRIMARY

us

us

us

us

SERVICE 

AREA 

2

PRIMARY

PRIMARY

PRIMARY

PRIMARY

 MAC 

Domain

SERVICE 

AREA 

3

PRIMARY

PRIMARY

PRIMARY

PRIMARY

 MAC 

Domain

SECONDARY

dsf

5

dsf

6

dsf

7

dsf

8

8 

CHANNEL BONDING

SECONDARY

SECONDARY

SECONDARY

us

us

us

us

us

us

us

us

 MAC 

Domain


The common feature in the above two examples is the ability to use all the line card resources.

2. Software features
2.7.  Dynamic Bandwidth Sharing

The above examples makes use of the Cisco ability to share bandwidth to provide an 8 channel bonded data service even though a 4 channel bonding data service is nominally deployed. 
Dynamic bandwidth sharing also allows the 4 channel bonding downstream to support DOCSIS 2.0 service with bandwidth being shared between the different types of operation of the downstream. Limits can be set in either case providing reserved bandwidth for narrow cast services and wideband or bonding operation.

http://www.cisco.com/en/US/docs/ios/cable/configuration/guide/cmts_dyn_bw_sharing.html
2.8. VDOC

VDOC provides a means to economically provide linear video or broadcast video to subscribers in a CMTS serving area.

The MC20x20V-20D deployment examples detailed above and copied following shows a shared 4 channel bonding group – this bonding group shared across the 4 service areas supported by the one line card.

[image: image7.wmf]Frequency

MC

20

x

20

V

-

20

D

LINE 

CARD

us

us

us

us

 MAC 

Domain

SERVICE 

AREA 

1

usf

1

usf

0

dsf

4

dsf

5

dsf

6

dsf

7

usf

2

usf

3

us

us

us

us

PRIMARY

PRIMARY

PRIMARY

PRIMARY

us

us

us

us

 MAC 

Domain

us

us

us

us

 MAC 

Domain

PRIMARY

PRIMARY

PRIMARY

PRIMARY

PRIMARY

PRIMARY

PRIMARY

PRIMARY

PRIMARY

PRIMARY

PRIMARY

PRIMARY

 MAC 

Domain

 MAC 

Domain

4 

CHANNEL BONDING

SHARED 

8 

CHANNEL BONDING

SHARED 

4 

CHANNEL BONDED IPTV

us

PRIMARY

DOCSIS 

2

.

0 

SUPPORT

us

SERVICE 

AREA 

2

us

us

SECONDARY

SECONDARY

SECONDARY

SECONDARY

SERVICE 

AREA 

3

SERVICE 

AREA 

4

dsf

1

dsf

2

dsf

3

dsf

4


The uBR10012 can be configured to support 8x4 modems to provide 4 channels of unicast high speed data and 4 channels of bonded broadcast data. 

The following figure shows this configuration in more detail at a system level:
[image: image8.png]Integrated %

or

Modular

B3-

HSD/VoIP/ "
Narrowcast IPTV Service Group 1

RF

Spanning\

Broadcast
IPTV

0

HSD/VolP/
Narrowcast IPTV

STB/PC STB/PC STB/PC




If the shared or spanned bonded downstream contains multiple IPTV streams, then an IPTV STB attached to a cable modem can potentially view any of the IPTV streams contained in the bonded 4 channel downstream. 

Such a configuration would be able to support approximately 50 standard definition IPTV Variable Bit Rate (VBR) streams with the CMTS now providing the STATMUX ability that is often associated with the MUX of MPEG streams into a downstream channel. The other big advantage of the CMTS based STATMUX of multiple streams into a bonded downstream of high data capacity is that more channels can be packed into the same bandwidth than would be achieved by packing them into N x discrete downstreams. i.e. a single downstream will carry approximately 10 SD IPTV streams and 4 such downstreams only 40 compared to 50 for the bonded channel..
Where a single 4 channel bonded downstream is not sufficient to provide the number of IPTV linear streams required, additional capacity can be added in the form of sharing multiple 4 channel bonded downstreams. This upgrade leverages the fact that 8x4 modems have two wideband tuners and effectively what happens is that one tuner is locked to providing 4 channel bonded data service whilst the second tuner can be reprogrammed using standard DOCSIS 3.0 protocols to tune to any 4 channel bonded channel as required. 


[image: image9.wmf]Frequency

MC

20

x

20

V

-

20

D

LINE 

CARD

us

us

us

us

 MAC 

Domain

SERVICE 

AREA 

1

usf

1

usf

0

dsf

1

dsf

2

dsf

3

dsf

4

usf

2

usf

3

us

us

us

us

PRIMARY

PRIMARY

PRIMARY

PRIMARY

us

us

us

us

 MAC 

Domain

 MAC 

Domain

PRIMARY

PRIMARY

PRIMARY

PRIMARY

PRIMARY

PRIMARY

PRIMARY

PRIMARY

 MAC 

Domain

 MAC 

Domain

4 

CHANNEL BONDING

SHARED 

8 

CHANNEL BONDING

SHARED 

8 

CHANNEL BONDED IPTV

us

PRIMARY

DOCSIS 

2

.

0 

SUPPORT

us

SERVICE 

AREA 

2

us

SECONDARY

SECONDARY

SECONDARY

SECONDARY

SERVICE 

AREA 

3

us

us

us

us

us

SECONDARY

SECONDARY

SECONDARY

SECONDARY

dsf

5

dsf

6

dsf

7

dsf

8

dsf

9

dsf

10

dsf

11

dsf

12


Such a dual bonded channel upgrade can be delivered in the above MC20X20V based configuration but consideration should be given to a M-CMTS solution if such a high capacity linear IPTV requirement is planned. In an M-CMTS solution a bonded downstream can be shared across an entire chassis – not just a single line card.
Regardless of the solution selected once multiple bonded downstream are provided the uBR10012 CMTS is smart enough to track via IGMP requests (as a result of STB channel change) what IPTV stream the subscriber wishes to watch and will reconfigure the modem to receive the requested bonded downstream. Thus more capacity than one shared bonded downstream can be effectively delivered to a subscriber. 

2.9. Powerboost

Allow subscribers to have access to additional downstream bandwidth for short time durations i.e. Improves the user experience – get a web page faster without affecting the AVERAGE download speed.

Powerboost can be configured by:
· CLI 

· Service Class Name 

· Per service flow in modem configuration file 

More information can be found here:

http://www.cisco.com/en/US/docs/ios/cable/command/reference/cbl_03_cable_d.html
2.10. Rate adapt

Under DOCSIS 2.0/3.0, a cable modem is not allowed to issue another request until an outstanding one has been granted. This leads to a delay in requesting further bandwidth until the outstanding request is received by the CMTS, then granted on a MAP sometime in the future. Maps are sent out every 2ms, so the turnaround time is roughly 4ms. 

The Cisco CMTS can provide increased upstream speed when the upstream is not busy by issuing more grants than requested by a modem thus allowing more data to be transferred. This feature is configured globally or on a per upstream basis using the command “rate-adapt” 

http://www.cisco.com/en/US/docs/ios/cable/configuration/guide/cmts_upstream_rate_adapt.html
2.11. Subscriber Traffic Management
As downstream data rates increase, the time period it takes for a subscriber to exceed download caps reduces - and may drop to such a short time that the operator's traditional billing methods will not catch this event and traditional rate limiting and / or warnings sent to the subscriber will be implemented too late. This is very relevant in regions where penalty rates apply once a download cap is exceeded. The STM feature will remove the need to worry about this and adds further value to Cisco CMTS. 

The Subscriber Traffic Management feature allows service providers to identify and control subscribers who exceed the maximum bandwidth allowed under their registered quality-of-service (QoS) profiles. This feature supplements current techniques such as Network-Based Application Recognition (NBAR) and access control lists (ACLs), to ensure that a minority of users do not consume a majority of the cable network's bandwidth. 

Current subscriber controls, such as NBAR and ACLs, examine all packets coming into the CMTS. These techniques can curb a large volume of problem traffic, but they are not as effective in dealing with the latest generation of peer-to-peer file-sharing applications that can place heavy demands on a network's available bandwidth. The Subscriber Traffic Management feature allows service providers to focus on a minority of potential problem users without impacting network performance or other users who are abiding by their service agreements. 

In addition, when a cable modem goes offline and remains offline for 24 hours, the Cisco CMTS deletes its service flow IDs from its internal databases, and also deletes the modem's traffic counters. This can allow some users to exceed their bandwidth limits, go offline, and come back online with new counters. 

The Subscriber Traffic Management feature helps to thwart these types of theft-of-service attacks by implementing a penalty period for cable modems that violate their service level agreements (SLAs). Even if the cable modem goes offline, its counters are still reset, and the CMTS continues to enforce the penalty period. 

http://www.cisco.com/en/US/docs/cable/cmts/feature/ubsubmon.html
3. Configuration

3.7. References

	[1] 


	http://www.cisco.com/en/US/docs/interfaces_modules/cable/broadband_processing_engines/ubr_mc20x20v/configuration/guide/mc20x20v_cfg.html


	[2]
	http://www.cisco.com/en/US/docs/ios/cable/configuration/guide/VDOC_dbc_feature_ps2209_TSD_Products_Configuration_Guide_Chapter.html



3.8. Concepts

http://www.cisco.com/en/US/docs/interfaces_modules/cable/broadband_processing_engines/ubr_mc20x20v/configuration/guide/mc20x20v_cfg.html
The configuration concepts are fundamentally the same as the SPA-24XDS-SFP downstream – i.e. allocation of the “downstream integrated-cable” ( for spa would be downstream modular-cable) to an “interface cable” defines a primary downstream and the wideband groups define the bonding groups and the fiber-node spec defines what a modem can “see” and what is advertised in the downstream MDD messaging.
What the MX20X20V hardware will look like:
!

card 1 4jacket-1

card 1/0 SPA-1XTENGE-XFP-V2

card 1/1 24rfchannel-spa-1

card 1/2 24rfchannel-spa-1

card 1/3 24rfchannel-spa-1

card 1/1 2cable-dtcc

card 2/1 2cable-dtcc

card 3 2jacket-1

card 3/0 24rfchannel-spa-1

card 3/1 24rfchannel-spa-1

card 6/0 5cable-mc520u-d

card 6/1 5cable-mc520h-d

card 8/1 ubr10k-clc-mc2020v license 20X20
cable admission-control preempt priority-voice

!

SPA based configuration:
controller Modular-Cable 3/1/0

 rf-channel 0 cable downstream channel-id 217

 rf-channel 1 cable downstream channel-id 218

 rf-channel 2 cable downstream channel-id 219

Compared to MC20X20V configuration:
controller Integrated-Cable 8/1/0

 rf-channel 0 cable downstream channel-id 80
 rf-channel 0 frequency 453000000 annex B modulation 256qam interleave 32

 no rf-channel 0 rf-shutdown

 rf-channel 1 cable downstream channel-id 81
 rf-channel 1 frequency 459000000 annex B modulation 256qam interleave 32

 no rf-channel 1 rf-shutdown

 rf-channel 2 cable downstream channel-id 82
 rf-channel 2 frequency 465000000 annex B modulation 256qam interleave 32

 no rf-channel 2 rf-shutdown

 rf-channel 3 cable downstream channel-id 83
 rf-channel 3 frequency 471000000 annex B modulation 256qam interleave 32

 no rf-channel 3 rf-shutdown
!

Upstream specs here:

interface cable 8/1/0
cable upstream max-ports 4

 cable upstream bonding-group 1

  upstream 0

  upstream 1

  upstream 2

  upstream 3

  attributes 800000AB

 cable upstream resiliency channel-down-detect 32

!

 cable upstream 0 connector 0

 cable upstream 0 frequency 14600000

 cable upstream 0 channel-width 6400000 6400000

 cable upstream 0 load-balance group 1

 cable upstream 0 docsis-mode atdma

 cable upstream 0 minislot-size 1

 cable upstream 0 range-backoff 3 6

 cable upstream 0 modulation-profile 224 225 226

 cable upstream 0 attribute-mask AB

 no cable upstream 0 shutdown

!

and wideband interface defined like this:

interface Wideband-Cable8/1/0:0

 cable bonding-group-id 1377

 cable rf-channel 0

 cable rf-channel 1

 cable rf-channel 2

 cable rf-channel 3

!
With the option of adding downstream from other controllers (MAC domains) on the same line card like this:

interface Wideband-Cable8/1/0:0

 cable bonding-group-id 1377

 cable rf-channel 0

 cable rf-channel 1

 cable rf-channel 2

 cable rf-channel 3
 cable rf-channel controller 4 channel 0 bandwidth-percent 1
 cable rf-channel controller 4 channel 1 bandwidth-percent 1
 cable rf-channel controller 4 channel 2 bandwidth-percent 1
 cable rf-channel controller 4 channel 3 bandwidth-percent 1
 cable downstream attribute-mask 800000AB

Primary 20x20 DS are defined like this

interface Cable8/1/0

 downstream integrated-cable 8/1/0:0 - defines one primary

 no cable packet-cache

 cable upstream max-ports 4

 cable upstream 0 connector 0

 cable upstream 0 channel-width 1600000 1600000

 cable upstream 0 docsis-mode tdma

 cable upstream 0 minislot-size 4

 cable upstream 0 range-backoff 3 6

 cable upstream 0 modulation-profile 21

 cable upstream 0 shutdown

For a fiber-node - the following will configure a 4 channel bonding group with the primary marked as above being downstream integrated-cable 8/1/0:0 due to the fact this rf-channel is defined under the interface cable 8/1/0 specification

cable fiber-node 1

  description Serving Area 1

  downstream Inrgrated-Cable 8/1/0 rf-channel 0-3

  upstream Cable 8/1 connector 0-3

4. Examples

4.7. A basic MC20x20V configuration. 

[image: image10.wmf]MC

20

x

20

V

-

20

D

LINE 

CARD

us

us

us

us

SERVICE 

AREA 

1

usf

1

usf

0

dsf

1

dsf

2

dsf

3

dsf

4

usf

2

usf

3

PRIMARY

PRIMARY

PRIMARY

PRIMARY

 MAC 

Domain

4 

CHANNEL BONDING

PRIMARY

DOCSIS 

2

.

0 

SUPPORT

us

us

us

us

SERVICE 

AREA 

2

PRIMARY

PRIMARY

PRIMARY

PRIMARY

 MAC 

Domain

us

us

us

us

SERVICE 

AREA 

3

PRIMARY

PRIMARY

PRIMARY

PRIMARY

 MAC 

Domain

us

us

us

us

SERVICE 

AREA 

4

PRIMARY

PRIMARY

PRIMARY

PRIMARY

 MAC 

Domain

us

us

us

us

SERVICE 

AREA 

5

PRIMARY

PRIMARY

PRIMARY

PRIMARY

 MAC 

Domain


controller integrated-cable 8/1/0

 rf-channel 0 cable downstream channel-id 80
 rf-channel 0 frequency 453000000 annex B modulation 256qam interleave 32

 no rf-channel 0 rf-shutdown

 rf-channel 1 cable downstream channel-id 81
 rf-channel 1 frequency 459000000 annex B modulation 256qam interleave 32

 no rf-channel 1 rf-shutdown

 rf-channel 2 cable downstream channel-id 82
 rf-channel 2 frequency 465000000 annex B modulation 256qam interleave 32

 no rf-channel 2 rf-shutdown

 rf-channel 3 cable downstream channel-id 83
 rf-channel 3 frequency 471000000 annex B modulation 256qam interleave 32

 no rf-channel 3 rf-shutdown
controller Integrated-Cable 8/1/1
 rf-channel 0 cable downstream channel-id 84
 rf-channel 0 frequency 453000000 annex B modulation 256qam interleave 32

 no rf-channel 0 rf-shutdown

 rf-channel 1 cable downstream channel-id 85
 rf-channel 1 frequency 459000000 annex B modulation 256qam interleave 32

 no rf-channel 1 rf-shutdown

 rf-channel 2 cable downstream channel-id 86
 rf-channel 2 frequency 465000000 annex B modulation 256qam interleave 32

 no rf-channel 2 rf-shutdown

 rf-channel 3 cable downstream channel-id 87
 rf-channel 3 frequency 471000000 annex B modulation 256qam interleave 32

 no rf-channel 3 rf-shutdown
controller Integrated-Cable 8/1/2

!as above but for incrementing channel-id
controller Integrated-Cable 8/1/3

!as above but for incrementing channel-id
controller Integrated-Cable 8/1/4

!as above but for incrementing channel-id
interface cable8/1/0
! define the primary capable downstream
downstream integrated-cable 8/1/0:0 

downstream integrated-cable 8/1/0:1 

downstream integrated-cable 8/1/0:2 

downstream integrated-cable 8/1/0:3 

! 

! set up the upstream bonding
!
cable upstream max-ports 4

 cable upstream bonding-group 1

  upstream 0

  upstream 1

  upstream 2

  upstream 3

  attributes 800000AB

 cable upstream resiliency channel-down-detect 32

!

 cable upstream 0 connector 0

 cable upstream 0 frequency 14600000

 cable upstream 0 channel-width 6400000 6400000

 cable upstream 0 load-balance group 1

 cable upstream 0 docsis-mode atdma

 cable upstream 0 minislot-size 1

 cable upstream 0 range-backoff 3 6

 cable upstream 0 modulation-profile 224 225 226

 cable upstream 0 attribute-mask AB

 no cable upstream 0 shutdown

!

 cable upstream 1 connector 1

 cable upstream 1 frequency 21000000

 cable upstream 1 channel-width 6400000 6400000

 cable upstream 1 load-balance group 1

 cable upstream 1 docsis-mode atdma

 cable upstream 1 minislot-size 1

 cable upstream 1 range-backoff 3 6

 cable upstream 1 modulation-profile 224 225 226

 cable upstream 1 attribute-mask AB

 no cable upstream 1 shutdown

!

 cable upstream 2 connector 2

 cable upstream 2 frequency 27400000

 cable upstream 2 channel-width 6400000 6400000

 cable upstream 2 load-balance group 1

 cable upstream 2 docsis-mode atdma

 cable upstream 2 minislot-size 1

 cable upstream 2 range-backoff 3 6

 cable upstream 2 modulation-profile 224 225 226

 cable upstream 2 attribute-mask AB

 no cable upstream 2 shutdown

!

 cable upstream 3 connector 3

 cable upstream 3 frequency 33800000

 cable upstream 3 channel-width 3200000 3200000

 cable upstream 3 load-balance group 1

 cable upstream 3 docsis-mode tdma-atdma

 cable upstream 3 minislot-size 2

 cable upstream 3 range-backoff 3 6

 cable upstream 3 modulation-profile 124 125 126

 cable upstream 3 attribute-mask AB

 no cable upstream 3 shutdown

!

 cable sid-cluster-switching max-request 0

 cable sid-cluster-switching max-outstanding-byte 0

 cable sid-cluster-switching max-total-byte 0

 cable sid-cluster-switching max-time 0

Future ( Need to detail what is happening here 
0 is the default condition though

!

interface cable8/1/1
! as above but upstream number incrementing
!

interface cable8/1/2
! as above but upstream number incrementing
!

interface cable8/1/3
! as above but upstream number incrementing
!

interface cable8/1/4
! as above but upstream number incrementing
interface integrated-cable 8/1/0:1

 description ** narrowband modems here

 cable bundle 1

 cable dynamic-bw-sharing

! this is a primary downstream and at least 4% needed for primary operation

 cable rf-bandwidth-percent 4

!

interface Integrated-Cable 8/1/0:2

 description ** narrowband modems here

 cable bundle 1

 cable dynamic-bw-sharing

! this is a primary downstream and at least 4% needed for primary operation

 cable rf-bandwidth-percent 4

!

interface Integrated-Cable 8/1/0:3

 description ** narrowband modems here

 cable bundle 1

 cable dynamic-bw-sharing

! this is a primary downstream and at least 4% needed for primary operation

 cable rf-bandwidth-percent 4

!

interface Integrated-Cable 8/1/0:4

 description ** narrowband modems here

 cable bundle 1

 cable dynamic-bw-sharing

! this is a primary downstream and at least 4% needed for primary operation

 cable rf-bandwidth-percent 4

!

interface Wideband-Cable8/1/0:0

 cable bonding-group-id 1377
 cable dynamic-bw-sharing

! all rf-channel primary so we must specify bandwidth percent

! if not then 100% assumed for bonding
cable rf-channel 0 bandwidth percent 1
 cable rf-channel 1 bandwidth percent 1
 cable rf-channel 2 bandwidth percent 1
 cable rf-channel 3 bandwidth percent 1
 cable downstream attribute-mask 800000AB

Future ( Detail why attribute mask being set

!

interface Wideband-Cable8/1/1:0

 cable bonding-group-id 1378
 cable rf-channel 0

 cable rf-channel 1

 cable rf-channel 2

 cable rf-channel 3
 cable downstream attribute-mask 800000AB

!

interface Wideband-Cable8/1/2:0

! as above but incrementing the bonding-group-id

!

interface Wideband-Cable8/1/3:0

! as above but incrementing the bonding-group-id
!

interface Wideband-Cable8/1/4:0

! as above but incrementing the bonding-group-id
cable fiber-node 1

  description Serving Area 1

  downstream Integrated-Cable 8/1/0 rf-channel 0-3

  upstream Cable 8/1 connector 0-3

!

cable fiber-node 2
  description Serving Area 2
  downstream Integrated-Cable 8/1/1 rf-channel 0-3

  upstream Cable 8/1 connector 4-7
!

cable fiber-node 3
  description Serving Area 3
  downstream Integrated-Cable 8/1/1 rf-channel 0-3

  upstream Cable 8/1 connector 8-11
!

cable fiber-node 4
  description Serving Area 4
  downstream Integrated-Cable 8/1/1 rf-channel 0-3

  upstream Cable 8/1 connector 12-15
!

cable fiber-node 5
  description Serving Area 5
  downstream Integrated-Cable 8/1/1 rf-channel 0-3

  upstream Cable 8/1 connector 16-19
4.8. A basic MC88V configuration. 

Work in progress

[image: image11.wmf]MC

20

x

20

V

-

20

D

LINE 

CARD

us

us

us

us

SERVICE 

AREA 

1

usf

1

usf

0

dsf

1

dsf

2

dsf

3

dsf

4

usf

2

usf

3

PRIMARY

PRIMARY

PRIMARY

PRIMARY

 MAC 

Domain

4 

CHANNEL BONDING

PRIMARY

us

us

us

us

SERVICE 

AREA 

2

PRIMARY

PRIMARY

PRIMARY

PRIMARY

 MAC 

Domain


controller integrated-cable 8/1/0

 rf-channel 0 cable downstream channel-id 80
 rf-channel 0 frequency 453000000 annex B modulation 256qam interleave 32

 no rf-channel 0 rf-shutdown

 rf-channel 1 cable downstream channel-id 81
 rf-channel 1 frequency 459000000 annex B modulation 256qam interleave 32

 no rf-channel 1 rf-shutdown

 rf-channel 2 cable downstream channel-id 82
 rf-channel 2 frequency 465000000 annex B modulation 256qam interleave 32

 no rf-channel 2 rf-shutdown

 rf-channel 3 cable downstream channel-id 83
 rf-channel 3 frequency 471000000 annex B modulation 256qam interleave 32

 no rf-channel 3 rf-shutdown
controller Integrated-Cable 8/1/1
 rf-channel 0 cable downstream channel-id 84
 rf-channel 0 frequency 453000000 annex B modulation 256qam interleave 32

 no rf-channel 0 rf-shutdown

 rf-channel 1 cable downstream channel-id 85
 rf-channel 1 frequency 459000000 annex B modulation 256qam interleave 32

 no rf-channel 1 rf-shutdown

 rf-channel 2 cable downstream channel-id 86
 rf-channel 2 frequency 465000000 annex B modulation 256qam interleave 32

 no rf-channel 2 rf-shutdown

 rf-channel 3 cable downstream channel-id 87
 rf-channel 3 frequency 471000000 annex B modulation 256qam interleave 32

 no rf-channel 3 rf-shutdown
controller Integrated-Cable 8/1/2

!as above but for incrementing channel-id
controller Integrated-Cable 8/1/3

!as above but for incrementing channel-id
controller Integrated-Cable 8/1/4

!as above but for incrementing channel-id
interface cable8/1/0
! define the primary capable downstream

downstream integrated-cable 8/1/0:0 

downstream integrated-cable 8/1/0:1 

downstream integrated-cable 8/1/0:2 

downstream integrated-cable 8/1/0:3 

! 

! set up the upstream bonding

!
cable upstream max-ports 4

 cable upstream bonding-group 1

  upstream 0

  upstream 1

  upstream 2

  upstream 3

  attributes 800000AB

 cable upstream resiliency channel-down-detect 32

!

 cable upstream 0 connector 0

 cable upstream 0 frequency 14600000

 cable upstream 0 channel-width 6400000 6400000

 cable upstream 0 load-balance group 1

 cable upstream 0 docsis-mode atdma

 cable upstream 0 minislot-size 1

 cable upstream 0 range-backoff 3 6

 cable upstream 0 modulation-profile 224 225 226

 cable upstream 0 attribute-mask AB

 no cable upstream 0 shutdown

!

 cable upstream 1 connector 1

 cable upstream 1 frequency 21000000

 cable upstream 1 channel-width 6400000 6400000

 cable upstream 1 load-balance group 1

 cable upstream 1 docsis-mode atdma

 cable upstream 1 minislot-size 1

 cable upstream 1 range-backoff 3 6

 cable upstream 1 modulation-profile 224 225 226

 cable upstream 1 attribute-mask AB

 no cable upstream 1 shutdown

!

 cable upstream 2 connector 2

 cable upstream 2 frequency 27400000

 cable upstream 2 channel-width 6400000 6400000

 cable upstream 2 load-balance group 1

 cable upstream 2 docsis-mode atdma

 cable upstream 2 minislot-size 1

 cable upstream 2 range-backoff 3 6

 cable upstream 2 modulation-profile 224 225 226

 cable upstream 2 attribute-mask AB

 no cable upstream 2 shutdown

!

 cable upstream 3 connector 3

 cable upstream 3 frequency 33800000

 cable upstream 3 channel-width 3200000 3200000

 cable upstream 3 load-balance group 1

 cable upstream 3 docsis-mode tdma-atdma

 cable upstream 3 minislot-size 2

 cable upstream 3 range-backoff 3 6

 cable upstream 3 modulation-profile 124 125 126

 cable upstream 3 attribute-mask AB

 no cable upstream 3 shutdown

!

 cable sid-cluster-switching max-request 0

 cable sid-cluster-switching max-outstanding-byte 0

 cable sid-cluster-switching max-total-byte 0

 cable sid-cluster-switching max-time 0

Future ( Need to detail what is happening here 

0 is the default condition though

!

interface cable8/1/1
! as above but upstream number incrementing
!

interface cable8/1/2
! as above but upstream number incrementing
!

interface cable8/1/3
! as above but upstream number incrementing
!

interface cable8/1/4
! as above but upstream number incrementing
interface Wideband-Cable8/1/0:0

 cable bonding-group-id 1377

 cable rf-channel 0

 cable rf-channel 1

 cable rf-channel 2

 cable rf-channel 3
 cable downstream attribute-mask 800000AB

Future ( Detail why attribute mask being set

!

interface Wideband-Cable8/1/1:0

 cable bonding-group-id 1378
 cable rf-channel 0

 cable rf-channel 1

 cable rf-channel 2

 cable rf-channel 3
 cable downstream attribute-mask 800000AB

!

interface Wideband-Cable8/1/2:0

! as above but incrementing the bonding-group-id

!

interface Wideband-Cable8/1/3:0

! as above but incrementing the bonding-group-id
!

interface Wideband-Cable8/1/4:0

! as above but incrementing the bonding-group-id
cable fiber-node 1

  description Serving Area 1

  downstream Integrated-Cable 8/1/0 rf-channel 0-3

  upstream Cable 8/1 connector 0-3

!

cable fiber-node 2
  description Serving Area 2
  downstream Integrated-Cable 8/1/1 rf-channel 0-3

  upstream Cable 8/1 connector 4-7
!

cable fiber-node 3
  description Serving Area 3
  downstream Integrated-Cable 8/1/1 rf-channel 0-3

  upstream Cable 8/1 connector 8-11
!

cable fiber-node 4
  description Serving Area 4
  downstream Integrated-Cable 8/1/1 rf-channel 0-3

  upstream Cable 8/1 connector 12-15
!

cable fiber-node 5
  description Serving Area 5
  downstream Integrated-Cable 8/1/1 rf-channel 0-3

  upstream Cable 8/1 connector 16-19
4.9. Shared (spanned) 8 channel bonding into 4 service areas
This example shows how to allocate downstream from a controller to a bonding group in more than one MAC domain.
This makes the 20 downstream line card “look like” a 32 downstream line card and provides increased peak bandwidth in each downstream service area but ultimately the sustained throughput is that of a 20 downstream line card

This example can be used to inject the same multicast traffic into the 4 service areas without multicast replication being required.

WARNING: If the modem does not see contiguous 8 channels it will not come w-online without you defining a custom RCP in the CMTS.

[image: image12.wmf]Frequency

MC

20

x

20

V

-

20

D

LINE 

CARD

us

us

us

us

 MAC 

Domain

SERVICE 

AREA 

1

usf

1

usf

0

dsf

4

dsf

5

dsf

6

dsf

7

usf

2

usf

3

us

us

us

us

PRIMARY

PRIMARY

PRIMARY

PRIMARY

us

us

us

us

 MAC 

Domain

us

us

us

us

 MAC 

Domain

PRIMARY

PRIMARY

PRIMARY

PRIMARY

PRIMARY

PRIMARY

PRIMARY

PRIMARY

PRIMARY

PRIMARY

PRIMARY

PRIMARY

 MAC 

Domain

 MAC 

Domain

4 

CHANNEL BONDING

SHARED 

8 

CHANNEL BONDING

SHARED 

4 

CHANNEL BONDED IPTV

us

PRIMARY

DOCSIS 

2

.

0 

SUPPORT

us

SERVICE 

AREA 

2

us

us

SECONDARY

SECONDARY

SECONDARY

SECONDARY

SERVICE 

AREA 

3

SERVICE 

AREA 

4

dsf

1

dsf

2

dsf

3

dsf

4



[image: image13.wmf]Frequency

MC

20

x

20

V

-

20

D

LINE 

CARD

us

us

us

us

 MAC 

Domain

SERVICE 

AREA 

1

usf

1

usf

0

dsf

4

dsf

5

dsf

6

dsf

7

usf

2

usf

3

us

us

us

us

PRIMARY

PRIMARY

PRIMARY

PRIMARY

us

us

us

us

 MAC 

Domain

us

us

us

us

 MAC 

Domain

PRIMARY

PRIMARY

PRIMARY

PRIMARY

PRIMARY

PRIMARY

PRIMARY

PRIMARY

PRIMARY

PRIMARY

PRIMARY

PRIMARY

 MAC 

Domain

 MAC 

Domain

4 

CHANNEL BONDING

SHARED 

8 

CHANNEL BONDING

SHARED 

4 

CHANNEL BONDED IPTV

us

PRIMARY

DOCSIS 

2

.

0 

SUPPORT

us

SERVICE 

AREA 

2

us

us

SECONDARY

SECONDARY

SECONDARY

SECONDARY

SERVICE 

AREA 

3

SERVICE 

AREA 

4

dsf

1

dsf

2

dsf

3

dsf

4

TX

0

TX

1

TX

2

TX

3

TX

4

MC

20

x

20

V

-

20

D

Service Area

Service Area

Service Area

Service Area


controller integrated-cable 8/1/0

 rf-channel 0 cable downstream channel-id 80
 rf-channel 0 frequency 453000000 annex B modulation 256qam interleave 32

 no rf-channel 0 rf-shutdown

 rf-channel 1 cable downstream channel-id 81
 rf-channel 1 frequency 459000000 annex B modulation 256qam interleave 32

 no rf-channel 1 rf-shutdown

 rf-channel 2 cable downstream channel-id 82
 rf-channel 2 frequency 465000000 annex B modulation 256qam interleave 32

 no rf-channel 2 rf-shutdown

 rf-channel 3 cable downstream channel-id 83
 rf-channel 3 frequency 471000000 annex B modulation 256qam interleave 32

 no rf-channel 3 rf-shutdown
controller Integrated-Cable 8/1/1
 rf-channel 0 cable downstream channel-id 84
 rf-channel 0 frequency 453000000 annex B modulation 256qam interleave 32

 no rf-channel 0 rf-shutdown

 rf-channel 1 cable downstream channel-id 85
 rf-channel 1 frequency 459000000 annex B modulation 256qam interleave 32

 no rf-channel 1 rf-shutdown

 rf-channel 2 cable downstream channel-id 86
 rf-channel 2 frequency 465000000 annex B modulation 256qam interleave 32

 no rf-channel 2 rf-shutdown

 rf-channel 3 cable downstream channel-id 87
 rf-channel 3 frequency 471000000 annex B modulation 256qam interleave 32

 no rf-channel 3 rf-shutdown
controller Integrated-Cable 8/1/2

!as above but for incrementing channel-id
controller Integrated-Cable 8/1/3

!as above but for incrementing channel-id
controller Integrated-Cable 8/1/4

!as above but for incrementing channel-id
interface cable8/1/0
! define the primary capable downstream
downstream integrated-cable 8/1/0:0 

downstream integrated-cable 8/1/0:1 

downstream integrated-cable 8/1/0:2 

downstream integrated-cable 8/1/0:3 

!

cable upstream max-ports 5
 cable upstream bonding-group 1

  upstream 0

  upstream 1

  upstream 2

  upstream 3

  attributes 800000AB

 cable upstream resiliency channel-down-detect 32

!

 cable upstream 0 connector 0

 cable upstream 0 frequency 14600000

 cable upstream 0 channel-width 6400000 6400000

 cable upstream 0 load-balance group 1

 cable upstream 0 docsis-mode atdma

 cable upstream 0 minislot-size 1

 cable upstream 0 range-backoff 3 6

 cable upstream 0 modulation-profile 224 225 226

 cable upstream 0 attribute-mask AB

 no cable upstream 0 shutdown

!

 cable upstream 1 connector 1

 cable upstream 1 frequency 21000000

 cable upstream 1 channel-width 6400000 6400000

 cable upstream 1 load-balance group 1

 cable upstream 1 docsis-mode atdma

 cable upstream 1 minislot-size 1

 cable upstream 1 range-backoff 3 6

 cable upstream 1 modulation-profile 224 225 226

 cable upstream 1 attribute-mask AB

 no cable upstream 1 shutdown

!

 cable upstream 2 connector 2

 cable upstream 2 frequency 27400000

 cable upstream 2 channel-width 6400000 6400000

 cable upstream 2 load-balance group 1

 cable upstream 2 docsis-mode atdma

 cable upstream 2 minislot-size 1

 cable upstream 2 range-backoff 3 6

 cable upstream 2 modulation-profile 224 225 226

 cable upstream 2 attribute-mask AB

 no cable upstream 2 shutdown

!

 cable upstream 3 connector 3

 cable upstream 3 frequency 33800000

 cable upstream 3 channel-width 3200000 3200000

 cable upstream 3 load-balance group 1

 cable upstream 3 docsis-mode tdma-atdma

 cable upstream 3 minislot-size 2

 cable upstream 3 range-backoff 3 6

 cable upstream 3 modulation-profile 124 125 126

 cable upstream 3 attribute-mask AB

 no cable upstream 3 shutdown

!

 cable upstream 4 connector 4
 cable upstream 4 frequency 37000000

 cable upstream 4 channel-width 3200000 3200000

 cable upstream 4 load-balance group 1

 cable upstream 4 docsis-mode tdma-atdma

 cable upstream 4 minislot-size 2

 cable upstream 4 range-backoff 3 6

 cable upstream 4 modulation-profile 124 125 126

 cable upstream 4 attribute-mask AB

 no cable upstream 4 shutdown

!

cable sid-cluster-switching max-request 0

 cable sid-cluster-switching max-outstanding-byte 0

 cable sid-cluster-switching max-total-byte 0

 cable sid-cluster-switching max-time 0

!

interface cable8/1/1
! as above but upstream number incrementing
!

interface cable8/1/2
! as above but upstream number incrementing
!

interface cable8/1/3
! as above but upstream number incrementing
!

interface integrated-cable 8/1/0:0
 description ** narrowband modems here

 cable bundle 1

 cable dynamic-bw-sharing
! this is a primary downstream and at least 4% needed for primary operation

 cable rf-bandwidth-percent 4
!

interface Integrated-Cable 8/1/0:1
 description ** narrowband modems here

 cable bundle 1

 cable dynamic-bw-sharing
! this is a primary downstream and at least 4% needed for primary operation

 cable rf-bandwidth-percent 4
!

interface Integrated-Cable 8/1/0:2
 description ** narrowband modems here

 cable bundle 1

 cable dynamic-bw-sharing
! this is a primary downstream and at least 4% needed for primary operation

 cable rf-bandwidth-percent 4
!

interface Integrated-Cable 8/1/0:3
 description ** narrowband modems here

 cable bundle 1

 cable dynamic-bw-sharing
! this is a primary downstream and at least 4% needed for primary operation

 cable rf-bandwidth-percent 4
!

interface Wideband-Cable8/1/0:0

 cable bonding-group-id 1377
 cable dynamic-bw-sharing
 cable rf-channel 0 bandwidth-percent 1
 cable rf-channel 1 bandwidth-percent 1
 cable rf-channel 2 bandwidth-percent 1
 cable rf-channel 3 bandwidth-percent 1
 cable rf-channel controller 4 channel 0 bandwidth-percent 1
 cable rf-channel controller 4 channel 1 bandwidth-percent 1
 cable rf-channel controller 4 channel 2 bandwidth-percent 1
 cable rf-channel controller 4 channel 3 bandwidth-percent 1
 cable downstream attribute-mask 800000AB

!

interface Wideband-Cable8/1/1:0

 cable bonding-group-id 1378
 cable dynamic-bw-sharing
 cable rf-channel 0 bandwidth-percent 1
 cable rf-channel 1 bandwidth-percent 1
 cable rf-channel 2 bandwidth-percent 1
 cable rf-channel 3 bandwidth-percent 1
 cable rf-channel controller 4 channel 0 bandwidth-percent 1
 cable rf-channel controller 4 channel 1 bandwidth-percent 1
 cable rf-channel controller 4 channel 2 bandwidth-percent 1
 cable rf-channel controller 4 channel 3 bandwidth-percent 1
 cable downstream attribute-mask 800000AB

!

interface Wideband-Cable8/1/2:0

! as above but incrementing the bonding-group-id

!

interface Wideband-Cable8/1/3:0

! as above but incrementing the bonding-group-id
!

interface Wideband-Cable8/1/4:0
! this wideband group used for shared multicast traffic

! with 20 percent reserved for such mcast traffic

! NOTE: The RF channels used for secondary bonding groups are non-primary capable
! Reference [2]
 cable bonding-group-id 1381 secondary
 cable dynamic-bw-sharing
 cable rf-channel controller 4 channel 0 bandwidth-percent 20
 cable rf-channel controller 4 channel 1 bandwidth-percent 20
 cable rf-channel controller 4 channel 2 bandwidth-percent 20
 cable rf-channel controller 4 channel 3 bandwidth-percent 20
cable fiber-node 1

  description Serving Area 1

  downstream integrated-cable 8/1/0 rf-channel 0-3

  downstream integrated-cable 8/1/4 rf-channel 0-3
  upstream Cable 8/1 connector 0-3

!

cable fiber-node 2
  description Serving Area 2
downstream Integrated-Cable 8/1/1 rf-channel 0-3   

  downstream integrated-cable 8/1/4 rf-channel 0-3
  upstream Cable 8/1 connector 4-7
!

cable fiber-node 3
  description Serving Area 3
  downstream Integrated-Cable 8/1/2 rf-channel 0-3

  downstream integrated-cable 8/1/4 rf-channel 0-3
  upstream Cable 8/1 connector 8-11
!

cable fiber-node 4
  description Serving Area 4
  downstream Integrated-Cable 8/1/3 rf-channel 0-3

  downstream integrated-cable 8/1/4 rf-channel 0-3
  upstream Cable 8/1 connector 12-15
!

4.10. Shared (spanned) 4 channel IPTV into 4 service areas

This example shows how to use the downstream from a controller to provide dynamic linear (broadcast SDV ) IPTV into the 4 service areas supported by the line card,

WARNING: If the modem does not see contiguous 8 channels it will not come w-online without you defining a custom RCC-template in the CMTS.

[image: image14.wmf]Frequency

MC

20

x

20

V

-

20

D

LINE 

CARD

us

us

us

us

 MAC 

Domain

SERVICE 

AREA 

1

usf

1

usf

0

dsf

4

dsf

5

dsf

6

dsf

7

usf

2

usf

3

us

us

us

us

PRIMARY

PRIMARY

PRIMARY

PRIMARY

us

us

us

us

 MAC 

Domain

us

us

us

us

 MAC 

Domain

PRIMARY

PRIMARY

PRIMARY

PRIMARY

PRIMARY

PRIMARY

PRIMARY

PRIMARY

PRIMARY

PRIMARY

PRIMARY

PRIMARY

 MAC 

Domain

 MAC 

Domain

4 

CHANNEL BONDING

SHARED 

8 

CHANNEL BONDING

SHARED 

4 

CHANNEL BONDED IPTV

us

PRIMARY

DOCSIS 

2

.

0 

SUPPORT

us

SERVICE 

AREA 

2

us

us

SECONDARY

SECONDARY

SECONDARY

SECONDARY

SERVICE 

AREA 

3

SERVICE 

AREA 

4

dsf

1

dsf

2

dsf

3

dsf

4


4.10.1. Basic Multicast Forwarding to spanned bonded downstream
Please note that these commands are required to have CMTS forwarding dynamic Multicast packets.  However, BPI+, Authorization features are all optional for multicast packets to be forwarded correctly.
Reference 
Global Configuration Commands:

ip multicast-routing

ip multicast-routing [vrf]
! this is used for admission control and we are not doing that here
cable application-type 1 name vdoc

cable application-type 1 include multicast 65535

Multicast QOS

We need to define QOS as the QOS contains the ds attribute that we want to map the multicast to i.e. map the multicast to a bonded downstream with a matching attribute

cable service class 1 name MQOS_DEFAULT

cable service class 1 downstream

cable service class 1 max-rate 10000000

cable service class 1 min-rate 1000000

cable service class 2 name MQOS

cable service class 2 downstream

cable service class 2 max-rate 20000000

cable service class 2 min-rate 0

cable service class 2 req-attr-mask 800000FF

cable multicast group-qos default scn MQOS_DEFAULT aggregate

cable multicast group-qos 10 scn MQOS single

!

cable multicast qos group 20 priority 1  

 application-id 65535

 session-range 239.0.0.0 255.0.0.0

 group-qos 10

!

Define the RCC-template

Without this template, the multicast will not be mapped to the spanned bonded downstream. I this case I am using only one primary downstream and I need to make sure that it is defined as receive-channel 1

cable rcc-template 255

  rcp-id 00 10 18 80 81

  receive-module 1 first-center-frequency 359000000

receive-module 2 first-center-frequency 391000000

! note that modems expect a primary to be on the FIRST receive channel

  receive-channel 1 center-frequency 383000000 connected-receive-module 1 primary

  receive-channel 2 center-frequency 359000000 connected-receive-module 1

  receive-channel 3 center-frequency 367000000 connected-receive-module 1

  receive-channel 4 center-frequency 375000000 connected-receive-module 1

  receive-channel 5 center-frequency 391000000 connected-receive-module 2

  receive-channel 6 center-frequency 399000000 connected-receive-module 2

  receive-channel 7 center-frequency 407000000 connected-receive-module 2

  receive-channel 8 center-frequency 415000000 connected-receive-module 2

!

Interface Configuration Commands:

Cable interface

The cable interface hosting the spanned downstream. Each MAC domain (cable interface) hosting a spanned bonded downstream must contain the RCC-template. If you do not do this, the modems will come online quite ok if the downstream are all contiguous but when it comes to mapping the multicast to the secondary spanned bonded downstream, the spanned downstream will not be found. E.g. under debug you will see an error like 

“: No matching GC on WB:Wideband-Cable7/1/0:0.  Use Primary DS for (N/A,239.1.1.1)MCAST”
\

So use a config like this for each mac domain:
interface Cable7/1/0

 downstream Integrated-Cable 7/1/0 rf-channel 3

! not running upstream boding for now

 no cable mtc-mode

! each mac domain hosting the spanned interface MUST have the RCC-template defined in it

 cable rcc-template 255
 cable bundle 1

Wideband interfaces

In this case I am defining a dedicated 4 channel downstream bonding group to each mac domain and a shared 4 channel bonding group sourced from controller 7/1/4.

NOTE: the attribute tagging used. The 7/1/0:1 interface MUST have the same attribute as the QOS defined above.

interface Wideband-Cable7/1/0:0

 cable bundle 1

 cable bonding-group-id 710

 cable dynamic-bw-sharing

 cable rf-channel 0 bandwidth-percent 10

 cable rf-channel 1 bandwidth-percent 10

 cable rf-channel 2 bandwidth-percent 10

 cable rf-channel 3 bandwidth-percent 10

 cable downstream attribute-mask 800000AA

!

interface Wideband-Cable7/1/0:1

 cable bundle 1

 cable bonding-group-id 1058

 cable rf-channel controller 4 channel 0 bandwidth-percent 50

 cable rf-channel controller 4 channel 1 bandwidth-percent 50

 cable rf-channel controller 4 channel 2 bandwidth-percent 50

 cable rf-channel controller 4 channel 3 bandwidth-percent 50

 cable downstream attribute-mask 800000FF

!

Primary downstream

Each primary downstream must have enough bandwidth allocated here to allow DOCSIS MAC management  messages – at least 4%

In this case only one primary is defined.

interface Integrated-Cable7/1/0:3

 cable bundle 1

 cable rf-bandwidth-percent 50

Bundle Interface:

The bundle interface must include the multicast QOS group that defines (via the specified service calls name) the ATTRIBUTE to map for each of the expected multicast

interface Bundle1

 ip address 10.67.30.33 255.255.255.248 secondary

 ip address 10.250.0.1 255.255.255.0

 ip flow ingress

 ip pim sparse-mode

 ip igmp version 3

 cable multicast-qos group 20
 cable arp filter request-send 3 2

 cable arp filter reply-accept 3 2

 cable source-verify leasequery-filter upstream 2 5

 cable dhcp-parse option-43

 cable dhcp-parse option-60

 cable dhcp-giaddr policy

 cable helper-address 10.67.30.4

!

Gigabit Interface:

interface GigabitEthernet4/0/0

 ip address 10.67.30.2 255.255.255.224

 ip pim sparse-mode

 no negotiation auto

 cdp enable

FIber node:

cable fiber-node 10

  description Serving Area for 2020

  downstream Integrated-Cable 7/1/0 rf-channel 0-3

  downstream Integrated-Cable 7/1/4 rf-channel 0-3

  upstream Cable 7/1 connector 0

!

Multicast routing:

If the following debug shows the correct mapping of the multicast group to the spanned wideband interface nut you see no multicast traffic you may need something like this global configuration.

ip pim rp-address 10.67.30.24
Show Commands (debug):
To check that the multicast routing is ok using the following:

 uBR10k-DOCSIS-30#show ip mroute 

IP Multicast Routing Table

Flags: D - Dense, S - Sparse, B - Bidir Group, s - SSM Group, C - Connected,

       L - Local, P - Pruned, R - RP-bit set, F - Register flag,

       T - SPT-bit set, J - Join SPT, M - MSDP created entry, E - Extranet,

       X - Proxy Join Timer Running, A - Candidate for MSDP Advertisement,

       U - URD, I - Received Source Specific Host Report, 

       Z - Multicast Tunnel, z - MDT-data group sender, 

       Y - Joined MDT-data group, y - Sending to MDT-data group, 

       V - RD & Vector, v - Vector

Outgoing interface flags: H - Hardware switched, A - Assert winner

 Timers: Uptime/Expires

 Interface state: Interface, Next-Hop or VCD, State/Mode

(*, 239.255.255.250), 1w0d/00:02:56, RP 10.67.30.24, flags: SJC

  Incoming interface: GigabitEthernet4/0/0, RPF nbr 10.67.30.24

  Outgoing interface list:

    Bundle1, Forward/Sparse, 1w0d/00:02:56, H

(*, 239.0.0.1), 1w0d/stopped, RP 10.67.30.24, flags: SJCF

  Incoming interface: GigabitEthernet4/0/0, RPF nbr 10.67.30.24

  Outgoing interface list:

    Bundle1, Forward/Sparse, 00:00:08/00:02:56

(10.67.30.24, 239.0.0.1), 1w0d/00:02:59, flags: FT

  Incoming interface: GigabitEthernet4/0/0, RPF nbr 0.0.0.0, Registering

  Outgoing interface list:

    Bundle1, Forward/Sparse, 00:00:08/00:02:56, H
(*, 231.1.1.230), 1w0d/00:02:59, RP 10.67.30.24, flags: SJPC

  Incoming interface: GigabitEthernet4/0/0, RPF nbr 10.67.30.24

  Outgoing interface list: Null

(*, 224.117.80.75), 1w0d/00:02:55, RP 10.67.30.24, flags: SJPC

  Incoming interface: GigabitEthernet4/0/0, RPF nbr 10.67.30.24

  Outgoing interface list: Null

(*, 224.0.1.40), 1w1d/00:01:56, RP 10.67.30.24, flags: SJCL

  Incoming interface: GigabitEthernet4/0/0, RPF nbr 10.67.30.24

  Outgoing interface list:

    Bundle1, Forward/Sparse, 1w1d/00:01:56

uBR10k-DOCSIS-30#show ip igmp membership 

Flags: A  - aggregate, T - tracked

       L  - Local, S - static, V - virtual, R - Reported through v3 

       I - v3lite, U - Urd, M - SSM (S,G) channel 

       1,2,3 - The version of IGMP, the group is in

Channel/Group-Flags: 

       / - Filtering entry (Exclude mode (S,G), Include mode (G))

Reporter:

       <mac-or-ip-address> - last reporter if group is not explicitly tracked

       <n>/<m>      - <n> reporter in include mode, <m> reporter in exclude

 Channel/Group                  Reporter        Uptime   Exp.  Flags  Interface 

 *,239.255.255.250              10.67.30.34     1w1d     02:50 3A     Bu1

 *,239.0.0.1                    10.67.30.34     00:00:14 02:50 3A     Bu1
 *,231.1.1.230                  10.67.30.23     1w0d     02:53 2A     Gi4/0/0

 *,224.117.80.75                10.67.30.17     1w0d     02:49 2A     Gi4/0/0

 *,224.0.1.40                   10.250.0.1      1w1d     02:55 2LA    Bu1

uBR10k-DOCSIS-30#
To check that multicast traffic is actually flowing:

Examples of a debug session where we want to see multicast mapped to wideband 7/1/0:1 and then want to see traffic on this bonded channel:

uBR10k-DOCSIS-30#scm

                                                                               D

MAC Address    IP Address     I/F         MAC           Prim RxPwr  Timing Num I

                                          State         Sid  (dBmv) Offset CPE P

0022.ce7a.8328 10.250.0.3     C7/1/0/U0   w-online      1    0.00   629    0   N

0018.6812.9a00 10.250.0.93    C7/1/0/U0   online        2    -0.50  621    0   N

0022.ce7a.8352 10.250.0.4     C7/1/0/U0   w-online      3    0.00   629    0   N

0022.3ac8.459f 10.250.0.6     C7/1/0/U0   w-online      4    0.00   629    1   N

0022.ce7a.8334 10.250.0.5     C7/1/0/U0   w-online      5    0.00   629    0   N

0022.ce88.9cd6 10.250.0.9     C7/1/0/U0   online        6    0.50   258    0   N

0022.ce88.9cca 10.250.0.10    C7/1/0/U0   online        7    0.50   259    0   N

uBR10k-DOCSIS-30#

uBR10k-DOCSIS-30#show debug

CMTS:

  CMTS Multicast All debugging is on

CMTS specific:

  Debugging is on for Address 0022.3ac8.459f, Mask ffff.ffff.ffff (verbose)

uBR10k-DOCSIS-30#

uBR10k-DOCSIS-30#

uBR10k-DOCSIS-30#term mon

uBR10k-DOCSIS-30#

At this point start the multicast client (in this case VLC on windows XP so using IGMP v3)

uBR10k-DOCSIS-30#

uBR10k-DOCSIS-30#

*Nov 23 13:42:25.091: Get DS (m Bundle1, cm 0022.3ac8.459f, S N/A, G 239.0.0.1) selected Wideband-Cable7/1/0:1 match 1MCAST DCC: CM 0022.3ac8.459f is wideband online.Dcc will not be invoked

So we have a match and traffic is supposed to be forwarded to wideband 7/1/0:1 now but lets check this. We expect (in this case) to see an approx 12.5Mbps stream.

uBR10k-DOCSIS-30#

uBR10k-DOCSIS-30#term no mon

uBR10k-DOCSIS-30#

uBR10k-DOCSIS-30#show int wide 7/1/0:1 multicast-sessions 

Default Multicast Service Flow 3 on Wideband-Cable7/1/0:1 

Multicast Group    : 239.0.0.1
         Source    : N/A

         Act GCRs  : 1

         Interface : Bu1                   State: A      GI: Bu1         RC: 1  

         GCR       : GC   SAID    SFID    Key   GQC    GEn

                     20   8220    29      0     10     0    

Multicast Group    : 239.255.255.250

         Source    : N/A

         Act GCRs  : 1

         Interface : Bu1                   State: A      GI: Bu1         RC: 1  

         GCR       : GC   SAID    SFID    Key   GQC    GEn

                     20   8217    26      0     10     0    

uBR10k-DOCSIS-30#

uBR10k-DOCSIS-30#show int wideband 7/1/0:1 service-flow 29 qos

Sfid  Dir Curr  Sid   Sched  Prio MaxSusRate  MaxBrst     MinRsvRate  Throughput 

          State       Type

29    DS  act   8220  BE     0    20000000    3044        0           12920000   

uBR10k-DOCSIS-30#

uBR10k-DOCSIS-30#show int wideband 7/1/0:1 service-flow 29 counters

Sfid  Packets              Bytes                PacketDrop Bits/Sec   Packet/Sec

29    62448                85054176             0          12872000   1182      

uBR10k-DOCSIS-30#

uBR10k-DOCSIS-30#

4.11. Multicast BPI+

Important: Encrypted multicast over spanned downstream is NOT supported. This section is included for completeness only:

Multicast BPI+ basically reuses the Multicast QoS CLI model, under Group-Config(GC) object.
cable match address CLI has been deprecated and will not be allowed during runtime.  However, CMTS will accept this command on startup-config and auto-upgrade into new CLI.
Global Configuration Commands:

cable multicast group-encryption [group-encrypt-id] algorithm 56bit-des

cable multicast qos group [gc-id] priority [gc priority]

  session-range [group] [group-mask] [source] [source-mask]

  group-encryption [group-encrypt-id]

Interface Configuration Commands:

cable multicast-qos group [gc-id]

Configuration Example:

cable multicast group-encryption 30 algorithm 56bit-des

cable multicast qos group 40 priority 2

  session-range 230.0.0.0 255.0.0.0

  group-encryption 30

interface Cable5/0/0

  cable multicast-qos group 40

Show Commands:
Show interface [forwarding interface] multicast-session

Show interface [interface] key sid 0
4.12. Multicast Authorization

The Authorization is disabled by default and user is required to enable and configure this properly.

Global Configuration Commands:

Enable this feature globally:

cable multicast authorization enable default [default action- permit/deny] max-sessions [limit]

Configure authorization profiles and rules:

cable multicast authorization profile [profile-name]

  Match rule [ipv4/ipv6] [source prefix] [group prefix] 
priority [rule-priority] [action-permit/deny]

Configuration Example:

cable multicast auth enable default-action  deny  max-sessions 10

cable multicast auth profile GOLD 

   match rule ipv4 source 0.0.0.0/0 230.0.0.0/16 128 permit 

   match rule ipv4 source 10.1.1.1/8 232.0.0.0/8 128 permit 
end
CM Configuration is required to define with correct profile name, in this example, “GOLD”.

Show Commands:

show cable multicast authorization

show cable modem [mac/ip] verbose

  Display the associated profile and session limit for a CM

4.13. Static Multicast Session

This feature is always enabled and depending on CM Configuration file.   Users can use Show CLI to display the DSID for a particular session and CM from explicit tracking database.

Show Commands:

show cable multicast dsid

show cable multicast db

4.14. VIDEO server:
A suitable video server can be built using the VLC server/client application in a Windows XP or Linux environment.

The past demonstrations have used a Lenovo T61 laptop and is capable of generating 5 standard definition video streams.
The VLC server can be downloaded from:

http://www.videolan.org
Modify the batch file embedded following including vlc path, video file name and multicast address so that one-click can start the streaming


[image: image15.wmf]239.1.1.1.bat



 EMBED Package  [image: image16.wmf]239.1.1.2.bat


5. APPENDIX 1 – Spanned multicast forwarding running config
This config contained SPA and line card in slots 5 and 6 – I hope I have removed all related configuration leaving the relevant configuration
uBR10k-DOCSIS-30#show run

Building configuration...

Current configuration : 42664 bytes

!

version 12.2

no service pad

service timestamps debug datetime msec

service timestamps log datetime msec

no service password-encryption

service internal

!

hostname uBR10k-DOCSIS-30

!

boot-start-marker

boot system flash ubr10k4-k9p6u2-mz.122-33.SCD5.bin

boot-end-marker

!

logging queue-limit 100

no logging console

enable password cisco

!

no aaa new-model

facility-alarm outlet-temperature major 58

facility-alarm outlet-temperature minor 48

facility-alarm outlet-temperature critical 85

facility-alarm intake-temperature major 54

facility-alarm intake-temperature minor 45

facility-alarm intake-temperature critical 72

!

!

!

card 1 2jacket-1

card 1/1 2cable-dtcc

card 2/1 2cable-dtcc

card 4/0 1gigethernet-hh-1

card 4/1 1gigethernet-hh-1

card 7/1 ubr10k-clc-mc2020v license 20X20

cable application-type 1 name vdoc

cable application-type 1 include multicast 65535

cable admission-control preempt priority-voice

cable source-verify leasequery-filter downstream 10 5

!

!         

!

!

cable modem remote-query 15 public

cable spectrum-group 1 hop threshold 40

cable spectrum-group 1 band 19200000 37200000 6

cable modulation-profile 222 request 0 16 0 22 qpsk scrambler 152 no-diff 32 fixed qpsk0 1 2048  

cable modulation-profile 222 initial 5 34 0 48 16qam scrambler 152 no-diff 384 fixed qpsk1 1 2048  

cable modulation-profile 222 station 5 34 0 48 16qam scrambler 152 no-diff 384 fixed qpsk1 1 2048  

cable modulation-profile 222 a-short 6 76 6 22 32qam scrambler 152 no-diff 64 shortened qpsk1 1 2048  

cable modulation-profile 222 a-long 9 232 0 22 32qam scrambler 152 no-diff 64 shortened qpsk1 0 2048  

cable modulation-profile 222 a-ugs 9 232 0 22 16qam scrambler 152 no-diff 64 shortened qpsk1 1 2048  

cable modulation-profile 226 request 0 16 0 22 qpsk scrambler 152 no-diff 36 fixed qpsk0 1 2048  

cable modulation-profile 226 initial 5 34 0 48 qpsk scrambler 152 no-diff 98 fixed qpsk0 1 2048  

cable modulation-profile 226 station 5 34 0 48 qpsk scrambler 152 no-diff 98 fixed qpsk0 1 2048  

cable modulation-profile 226 a-short 6 76 6 22 64qam scrambler 152 no-diff 64 shortened qpsk1 1 2048  

cable modulation-profile 226 a-long 14 220 0 22 64qam scrambler 152 no-diff 64 shortened qpsk1 1 2048  

cable modulation-profile 226 a-ugs 14 220 0 22 64qam scrambler 152 no-diff 64 shortened qpsk1 1 2048  

cable modulation-profile 227 request 0 16 0 22 qpsk scrambler 152 no-diff 36 fixed qpsk0 1 2048  

cable modulation-profile 227 initial 5 34 0 48 qpsk scrambler 152 no-diff 98 fixed qpsk0 1 2048  

cable modulation-profile 227 station 5 34 0 48 qpsk scrambler 152 no-diff 98 fixed qpsk0 1 2048  

cable modulation-profile 227 a-short 4 76 7 22 16qam scrambler 152 no-diff 64 shortened qpsk1 1 2048  

cable modulation-profile 227 a-long 10 220 0 22 16qam scrambler 152 no-diff 64 shortened qpsk1 1 2048  

cable modulation-profile 227 a-ugs 10 220 0 22 16qam scrambler 152 no-diff 64 shortened qpsk1 1 2048  

cable service class 1 name MQOS_DEFAULT

cable service class 1 downstream

cable service class 1 max-rate 10000000

cable service class 1 min-rate 1000000

cable service class 2 name MQOS

cable service class 2 downstream

cable service class 2 max-rate 20000000

cable service class 2 min-rate 0

cable service class 2 req-attr-mask 800000FF

no cable qos permission create

no cable qos permission update

cable qos permission modems

cable multicast group-qos default scn MQOS_DEFAULT aggregate

cable multicast group-qos 10 scn MQOS single

!

cable multicast qos group 20 priority 1  

 application-id 65535

 session-range 239.0.0.0 255.0.0.0

 group-qos 10

!

cable time-server

!

!         

cable load-balance group 1

cable clock dti

!

!

cable rcc-template 255

  rcp-id 00 10 18 80 81

  receive-module 1 first-center-frequency 359000000

  receive-module 2 first-center-frequency 391000000

  receive-channel 1 center-frequency 383000000 connected-receive-module 1 primary

  receive-channel 2 center-frequency 359000000 connected-receive-module 1

  receive-channel 3 center-frequency 367000000 connected-receive-module 1

  receive-channel 4 center-frequency 375000000 connected-receive-module 1

  receive-channel 5 center-frequency 391000000 connected-receive-module 2

  receive-channel 6 center-frequency 399000000 connected-receive-module 2

  receive-channel 7 center-frequency 407000000 connected-receive-module 2

  receive-channel 8 center-frequency 415000000 connected-receive-module 2

!

ip subnet-zero

ip ftp username root

ip ftp password cisco

no ip domain lookup

no ip bootp server

ip multicast-routing 

!

!

!

!

!

!

access-list 110 permit igmp any 224.0.0.0 15.255.255.255

access-list 110 permit udp 172.17.1.0 0.0.0.255 host 211.29.80.16 eq snmp

access-list 110 permit udp 172.17.1.0 0.0.0.255 host 172.17.1.1 eq snmp

access-list 110 deny   ip any 224.0.0.0 15.255.255.255

access-list 110 deny   udp any any eq snmptrap

access-list 110 deny   tcp any any eq 161

access-list 110 deny   tcp any any eq 162

access-list 110 permit ip any any

access-list 112 permit tcp any any established

access-list 112 permit udp host 211.29.80.16 172.17.1.0 0.0.0.255 eq snmp

access-list 112 permit udp host 172.17.1.1 172.17.1.0 0.0.0.255 eq snmp

access-list 112 deny   tcp any any eq smtp

access-list 112 deny   tcp any any eq www

access-list 112 deny   tcp any any eq 1080

access-list 112 deny   tcp any any eq 445

access-list 112 deny   udp any any eq 445

access-list 112 deny   tcp any any eq 137

access-list 112 deny   tcp any any eq 138

access-list 112 deny   tcp any any eq 139

access-list 112 deny   udp any any eq netbios-ns

access-list 112 deny   udp any any eq netbios-dgm

access-list 112 deny   udp any any eq netbios-ss

access-list 112 deny   tcp any any eq 135

access-list 112 deny   udp any any eq 135

access-list 112 deny   tcp any any eq 593

access-list 112 permit ip any 10.0.0.0 0.255.255.255

access-list 112 deny   ip any 0.0.0.255 255.255.255.0

access-list 112 permit ip any any

!

redundancy

 mode sso

!

ip access-list standard public

!

!!

controller Integrated-Cable 7/1/0

 rf-channel 0 cable downstream channel-id 120

 rf-channel 0 frequency 359000000 annex A modulation 256qam interleave 12

 rf-channel 0 rf-power 52.0

 no rf-channel 0 rf-shutdown

 rf-channel 1 cable downstream channel-id 121

 rf-channel 1 frequency 367000000 annex A modulation 256qam interleave 12

 rf-channel 1 rf-power 52.0

 no rf-channel 1 rf-shutdown

 rf-channel 2 cable downstream channel-id 122

 rf-channel 2 frequency 375000000 annex A modulation 256qam interleave 12

 rf-channel 2 rf-power 52.0

 no rf-channel 2 rf-shutdown

 rf-channel 3 cable downstream channel-id 123

 rf-channel 3 frequency 383000000 annex A modulation 256qam interleave 12

 rf-channel 3 rf-power 52.0

 no rf-channel 3 rf-shutdown

!

controller Integrated-Cable 7/1/1

 rf-channel 0 cable downstream channel-id 125

 rf-channel 0 frequency none annex A modulation 256qam interleave 12

 rf-channel 0 rf-shutdown

 rf-channel 1 cable downstream channel-id 126

 rf-channel 1 frequency none annex A modulation 256qam interleave 12

 rf-channel 1 rf-shutdown

 rf-channel 2 cable downstream channel-id 127

 rf-channel 2 frequency none annex A modulation 256qam interleave 12

 rf-channel 2 rf-shutdown

 rf-channel 3 cable downstream channel-id 128

 rf-channel 3 frequency none annex A modulation 256qam interleave 12

 rf-channel 3 rf-shutdown

!

controller Integrated-Cable 7/1/2

 rf-channel 0 cable downstream channel-id 129

 rf-channel 0 frequency none annex A modulation 256qam interleave 12

 rf-channel 0 rf-shutdown

 rf-channel 1 cable downstream channel-id 130

 rf-channel 1 frequency none annex A modulation 256qam interleave 12

 rf-channel 1 rf-shutdown

 rf-channel 2 cable downstream channel-id 131

 rf-channel 2 frequency none annex A modulation 256qam interleave 12

 rf-channel 2 rf-shutdown

 rf-channel 3 cable downstream channel-id 132

 rf-channel 3 frequency none annex A modulation 256qam interleave 12

 rf-channel 3 rf-shutdown

!

controller Integrated-Cable 7/1/3

 rf-channel 0 cable downstream channel-id 133

 rf-channel 0 frequency none annex A modulation 256qam interleave 12

 rf-channel 0 rf-shutdown

 rf-channel 1 cable downstream channel-id 134

 rf-channel 1 frequency none annex A modulation 256qam interleave 12

 rf-channel 1 rf-shutdown

 rf-channel 2 cable downstream channel-id 135

 rf-channel 2 frequency none annex A modulation 256qam interleave 12

 rf-channel 2 rf-shutdown

 rf-channel 3 cable downstream channel-id 136

 rf-channel 3 frequency none annex A modulation 256qam interleave 12

 rf-channel 3 rf-shutdown

!

controller Integrated-Cable 7/1/4

 rf-channel 0 cable downstream channel-id 137

 rf-channel 0 frequency 391000000 annex A modulation 256qam interleave 12

 rf-channel 0 rf-power 52.0

 no rf-channel 0 rf-shutdown

 rf-channel 1 cable downstream channel-id 138

 rf-channel 1 frequency 399000000 annex A modulation 256qam interleave 12

 rf-channel 1 rf-power 52.0

 no rf-channel 1 rf-shutdown

 rf-channel 2 cable downstream channel-id 139

 rf-channel 2 frequency 407000000 annex A modulation 256qam interleave 12

 rf-channel 2 rf-power 52.0

 no rf-channel 2 rf-shutdown

 rf-channel 3 cable downstream channel-id 140

 rf-channel 3 frequency 415000000 annex A modulation 256qam interleave 12

 rf-channel 3 rf-power 52.0

 no rf-channel 3 rf-shutdown

!

! 

!

!

!

!

interface FastEthernet0/0/0

 no ip address

 no ip route-cache cef

 no ip route-cache

 shutdown

 media-type rj45

 speed 100

 full-duplex

!

interface GigabitEthernet1/0/0

 no ip address

 negotiation auto

 output-rate 10000

!

interface Wideband-Cable1/0/0:0

 cable bundle 1

 cable bonding-group-id 1

 cable rf-channel 0 bandwidth-percent 1

 cable rf-channel 1

 cable rf-channel 2

 cable rf-channel 3

!

interface Wideband-Cable1/0/0:1

 cable bundle 1

 cable bonding-group-id 2

 cable rf-channel 4 bandwidth-percent 1

 cable rf-channel 5

 cable rf-channel 6

 cable rf-channel 7

!

interface Modular-Cable1/0/0:0

 shutdown

 cable bundle 1

 cable rf-bandwidth-percent 95

!

interface Modular-Cable1/0/0:4

 cable bundle 1

 cable rf-bandwidth-percent 95

!

interface GigabitEthernet1/1/0

 no ip address

 negotiation auto

 output-rate 10000

!

interface GigabitEthernet4/0/0

 ip address 10.67.30.2 255.255.255.224

 ip pim sparse-mode

 no negotiation auto

 cdp enable

!

interface GigabitEthernet4/1/0

 no ip address

 negotiation auto

!

!

interface Cable7/1/0

 downstream Integrated-Cable 7/1/0 rf-channel 3

 no cable mtc-mode

 cable rcc-template 255

 no cable packet-cache

 cable bundle 1

 cable upstream max-ports 4

 cable upstream resiliency channel-down-detect 32

 cable upstream 0 connector 0

 cable upstream 0 frequency 14600000

 cable upstream 0 channel-width 6400000 6400000

 cable upstream 0 docsis-mode atdma

 cable upstream 0 minislot-size 1

 cable upstream 0 range-backoff 3 6

 cable upstream 0 modulation-profile 221

 no cable upstream 0 shutdown

 cable upstream 1 connector 1

 cable upstream 1 channel-width 1600000 1600000

 cable upstream 1 docsis-mode tdma

 cable upstream 1 minislot-size 4

 cable upstream 1 range-backoff 3 6

 cable upstream 1 modulation-profile 21

 cable upstream 1 shutdown

 cable upstream 2 connector 2

 cable upstream 2 channel-width 1600000 1600000

 cable upstream 2 docsis-mode tdma

 cable upstream 2 minislot-size 4

 cable upstream 2 range-backoff 3 6

 cable upstream 2 modulation-profile 21

 cable upstream 2 shutdown

 cable upstream 3 connector 3

 cable upstream 3 channel-width 1600000 1600000

 cable upstream 3 docsis-mode tdma

 cable upstream 3 minislot-size 4

 cable upstream 3 range-backoff 3 6

 cable upstream 3 modulation-profile 21

 cable upstream 3 shutdown

!

interface Wideband-Cable7/1/0:0

 cable bundle 1

 cable bonding-group-id 710

 cable dynamic-bw-sharing

 cable rf-channel 0 bandwidth-percent 10

 cable rf-channel 1 bandwidth-percent 10

 cable rf-channel 2 bandwidth-percent 10

 cable rf-channel 3 bandwidth-percent 10

 cable downstream attribute-mask 800000AA

!

interface Wideband-Cable7/1/0:1

 cable bundle 1

 cable bonding-group-id 1058

 cable rf-channel controller 4 channel 0 bandwidth-percent 50

 cable rf-channel controller 4 channel 1 bandwidth-percent 50

 cable rf-channel controller 4 channel 2 bandwidth-percent 50

 cable rf-channel controller 4 channel 3 bandwidth-percent 50

 cable downstream attribute-mask 800000FF

!

interface Integrated-Cable7/1/0:0

 cable rf-bandwidth-percent 50

!

interface Integrated-Cable7/1/0:1

 cable rf-bandwidth-percent 50

!

interface Integrated-Cable7/1/0:2

 cable rf-bandwidth-percent 50

!

interface Integrated-Cable7/1/0:3

 cable bundle 1

 cable rf-bandwidth-percent 50

!

interface Cable7/1/1

 no cable packet-cache

 cable upstream max-ports 4

 cable upstream 0 connector 4

 cable upstream 0 channel-width 1600000 1600000

 cable upstream 0 docsis-mode tdma

 cable upstream 0 minislot-size 4

 cable upstream 0 range-backoff 3 6

 cable upstream 0 modulation-profile 21

 cable upstream 0 shutdown

 cable upstream 1 connector 5

 cable upstream 1 channel-width 1600000 1600000

 cable upstream 1 docsis-mode tdma

 cable upstream 1 minislot-size 4

 cable upstream 1 range-backoff 3 6

 cable upstream 1 modulation-profile 21

 cable upstream 1 shutdown

 cable upstream 2 connector 6

 cable upstream 2 channel-width 1600000 1600000

 cable upstream 2 docsis-mode tdma

 cable upstream 2 minislot-size 4

 cable upstream 2 range-backoff 3 6

 cable upstream 2 modulation-profile 21

 cable upstream 2 shutdown

 cable upstream 3 connector 7

 cable upstream 3 channel-width 1600000 1600000

 cable upstream 3 docsis-mode tdma

 cable upstream 3 minislot-size 4

 cable upstream 3 range-backoff 3 6

 cable upstream 3 modulation-profile 21

 cable upstream 3 shutdown

!

interface Cable7/1/2

 no cable packet-cache

 cable upstream max-ports 4

 cable upstream 0 connector 8

 cable upstream 0 channel-width 1600000 1600000

 cable upstream 0 docsis-mode tdma

 cable upstream 0 minislot-size 4

 cable upstream 0 range-backoff 3 6

 cable upstream 0 modulation-profile 21

 cable upstream 0 shutdown

 cable upstream 1 connector 9

 cable upstream 1 channel-width 1600000 1600000

 cable upstream 1 docsis-mode tdma

 cable upstream 1 minislot-size 4

 cable upstream 1 range-backoff 3 6

 cable upstream 1 modulation-profile 21

 cable upstream 1 shutdown

 cable upstream 2 connector 10

 cable upstream 2 channel-width 1600000 1600000

 cable upstream 2 docsis-mode tdma

 cable upstream 2 minislot-size 4

 cable upstream 2 range-backoff 3 6

 cable upstream 2 modulation-profile 21

 cable upstream 2 shutdown

 cable upstream 3 connector 11

 cable upstream 3 channel-width 1600000 1600000

 cable upstream 3 docsis-mode tdma

 cable upstream 3 minislot-size 4

 cable upstream 3 range-backoff 3 6

 cable upstream 3 modulation-profile 21

 cable upstream 3 shutdown

!

interface Cable7/1/3

 no cable packet-cache

 cable upstream max-ports 4

 cable upstream 0 connector 12

 cable upstream 0 channel-width 1600000 1600000

 cable upstream 0 docsis-mode tdma

 cable upstream 0 minislot-size 4

 cable upstream 0 range-backoff 3 6

 cable upstream 0 modulation-profile 21

 cable upstream 0 shutdown

 cable upstream 1 connector 13

 cable upstream 1 channel-width 1600000 1600000

 cable upstream 1 docsis-mode tdma

 cable upstream 1 minislot-size 4

 cable upstream 1 range-backoff 3 6

 cable upstream 1 modulation-profile 21

 cable upstream 1 shutdown

 cable upstream 2 connector 14

 cable upstream 2 channel-width 1600000 1600000

 cable upstream 2 docsis-mode tdma

 cable upstream 2 minislot-size 4

 cable upstream 2 range-backoff 3 6

 cable upstream 2 modulation-profile 21

 cable upstream 2 shutdown

 cable upstream 3 connector 15

 cable upstream 3 channel-width 1600000 1600000

 cable upstream 3 docsis-mode tdma

 cable upstream 3 minislot-size 4

 cable upstream 3 range-backoff 3 6

 cable upstream 3 modulation-profile 21

 cable upstream 3 shutdown

!

interface Cable7/1/4

 no cable packet-cache

 cable upstream max-ports 4

 cable upstream 0 connector 16

 cable upstream 0 channel-width 1600000 1600000

 cable upstream 0 docsis-mode tdma

 cable upstream 0 minislot-size 4

 cable upstream 0 range-backoff 3 6

 cable upstream 0 modulation-profile 21

 cable upstream 0 shutdown

 cable upstream 1 connector 17

 cable upstream 1 channel-width 1600000 1600000

 cable upstream 1 docsis-mode tdma

 cable upstream 1 minislot-size 4

 cable upstream 1 range-backoff 3 6

 cable upstream 1 modulation-profile 21

 cable upstream 1 shutdown

 cable upstream 2 connector 18

 cable upstream 2 channel-width 1600000 1600000

 cable upstream 2 docsis-mode tdma

 cable upstream 2 minislot-size 4

 cable upstream 2 range-backoff 3 6

 cable upstream 2 modulation-profile 21

 cable upstream 2 shutdown

 cable upstream 3 connector 19

 cable upstream 3 channel-width 1600000 1600000

 cable upstream 3 docsis-mode tdma

 cable upstream 3 minislot-size 4

 cable upstream 3 range-backoff 3 6

 cable upstream 3 modulation-profile 21

 cable upstream 3 shutdown

!

interface Bundle1

 ip address 10.67.30.33 255.255.255.248 secondary

 ip address 10.250.0.1 255.255.255.0

 ip flow ingress

 ip pim sparse-mode

 ip igmp version 3

 cable multicast-qos group 20

 cable arp filter request-send 3 2

 cable arp filter reply-accept 3 2

 cable source-verify leasequery-filter upstream 2 5

 cable dhcp-parse option-43

 cable dhcp-parse option-60

 cable dhcp-giaddr policy

 cable helper-address 10.67.30.4

!

!

ip classless

ip route 0.0.0.0 0.0.0.0 10.67.30.1

ip route 10.250.0.0 255.255.255.0 10.81.88.138

!

!

no ip http server

no ip http secure-server

ip pim rp-address 10.67.30.24
!

ip explicit-path name test enable

!

kron occurrence kron-do-tdbg in 5 recurring

 policy-list do-tdbg

!

logging cmts cr10k log-level errors

logging cmts sea syslog-level errors

cpd cr-id 1

nls resp-timeout 1

cdp run

!

tftp-server bootflash:docsis11.cm

tftp-server bootflash:epc3925-ESIP-11-v302r12911-100209c-TLS-CVC060414.p7b

tftp-server bootflash:sa_upgrade-rpage-epc3925.cm

snmp-server community private RW

snmp-server location Melbourne

snmp-server contact Rob Page

snmp-server manager

snmp ifmib ifindex persist

!

!

control-plane

!

alias exec scm show cable modem

alias exec ccmad clear cable modem all delete

!

line con 0

 stopbits 1

line aux 0

 stopbits 1

line vty 0 4

 exec-timeout 0 0

 password cisco

 login

line vty 5 10

 password cisco

 login

!

!

cable fiber-node 10

  description Serving Area for 2020

  downstream Integrated-Cable 7/1/0 rf-channel 0-3

  downstream Integrated-Cable 7/1/4 rf-channel 0-3

  upstream Cable 7/1 connector 0

!

event manager applet CPULOAD 

 event snmp oid "1.3.6.1.4.1.9.9.109.1.1.1.1.8.1" get-type exact entry-op ge entry-val "75" poll-interval 5

 action 1 cli command "show process cpu sort 5s | ex 0.00%"

!

end

uBR10k-DOCSIS-30#           

6. APPENDIX 2 - UBR10012 configuration sample 1 (VDOC)
At  present this is just a CLI listing of the running config THAT DOES NOT USE THE MC20x20V with relevant sections highlighted.
Future ( to be updated for MC20x20V configuration
Yellow is the CLI for VDOC.
http://www.cisco.com/en/US/docs/ios/cable/configuration/guide/VDOC_dbc_feature.html
Green is the CLI for MQoS.
http://www.cisco.com/en/US/docs/ios/cable/configuration/guide/ubr_d30_mcast_support.html#wp1083525
uBR10K#

uBR10K#

uBR10K#sh run

Building configuration...

Current configuration : 21095 bytes

!

! Last configuration change at 11:16:09 UTC Sun Apr 1 2001

! NVRAM config last updated at 14:58:55 UTC Sun Apr 1 2001

!

version 12.2

no service pad

service timestamps debug datetime msec localtime

service timestamps log datetime msec localtime

no service password-encryption

!

hostname uBR10K

!

boot-start-marker

boot system flash disk0:ubr10k2-k9p6u2-mz.122-33.SCB.bin

boot-end-marker

!

logging buffered 1000000 informational

enable password cisco

!

no aaa new-model

clock calendar-valid

facility-alarm core-temperature major 58

facility-alarm core-temperature minor 50

facility-alarm intake-temperature major 54

facility-alarm intake-temperature minor 45

!

!

cr10k-rp card 5/0 oir-compatibility

cr10k-rp card 6/1 oir-compatibility

card 1 2jacket-1

card 1/0 24rfchannel-spa-1

card 1/1 2cable-dtcc

card 2/1 2cable-dtcc

card 4/0 1gigethernet-hh-1

card 4/1 1gigethernet-hh-1

card 6/1 5cable-mc520h-d

cable admission-control preempt priority-voice

cable modem remote-query 10 public

cable modem vendor 00.17.EE "Motorola"

cable modem vendor 00.18.68 "SA"

cable modem vendor 00.16.92 "SA"

cable modem vendor 00.40.7B "SA"

cable modem vendor 00.1A.C3 "Cisco"

cable modem vendor 00.14.E8 "Motorola"

cable modem vendor 00.21.43 "Motorola"

cable modem vendor 00.1F.C4 "Motorola"

cable modulation-profile 222 request 0 16 0 22 64qam scrambler 152 no-diff 64 fixed qpsk1 1 2048  

cable modulation-profile 222 initial 5 34 0 48 64qam scrambler 152 no-diff 128 fixed qpsk1 1 2048  

cable modulation-profile 222 station 5 34 0 48 64qam scrambler 152 no-diff 128 fixed qpsk1 1 2048  

cable modulation-profile 222 a-short 6 76 6 22 64qam scrambler 152 no-diff 64 shortened qpsk1 1 2048  

cable modulation-profile 222 a-long 9 232 0 22 64qam scrambler 152 no-diff 64 shortened qpsk1 1 2048  

cable modulation-profile 222 a-ugs 9 232 0 22 64qam scrambler 152 no-diff 64 shortened qpsk1 1 2048  

cable modulation-profile 223 request 0 16 0 22 64qam scrambler 152 no-diff 64 fixed qpsk1 1 2048  

cable modulation-profile 223 initial 5 34 0 48 64qam scrambler 152 no-diff 128 fixed qpsk1 1 2048  

cable modulation-profile 223 station 5 34 0 48 64qam scrambler 152 no-diff 128 fixed qpsk1 1 2048  

cable modulation-profile 223 a-short 6 76 6 22 64qam scrambler 152 no-diff 64 shortened qpsk1 1 2048  

cable modulation-profile 223 a-long 9 232 0 22 64qam scrambler 152 no-diff 64 shortened qpsk1 1 2048  

cable modulation-profile 223 a-ugs 9 232 0 22 64qam scrambler 152 no-diff 64 shortened qpsk1 1 2048  

cable modulation-profile 224 request 0 16 0 22 16qam scrambler 152 no-diff 32 fixed qpsk1 1 2048  

cable modulation-profile 224 initial 5 34 0 48 16qam scrambler 152 no-diff 64 fixed qpsk1 1 2048  

cable modulation-profile 224 station 5 34 0 48 16qam scrambler 152 no-diff 64 fixed qpsk1 1 2048  

cable modulation-profile 224 a-short 4 76 7 22 16qam scrambler 152 no-diff 64 shortened qpsk1 1 2048  

cable modulation-profile 224 a-long 9 232 0 22 16qam scrambler 152 no-diff 64 shortened qpsk1 1 2048  

cable modulation-profile 224 a-ugs 9 232 0 22 16qam scrambler 152 no-diff 64 shortened qpsk1 1 2048  

cable service class 1 name def_sclass

cable service class 1 downstream

cable service class 1 max-rate 10000000

cable service class 1 min-rate 0

cable service class 5 name MQoS

cable service class 5 downstream

cable service class 5 max-rate 38000000

cable service class 5 max-burst 250000

cable service class 5 min-rate 10000000

cable service class 5 priority 4
cable service attribute ds-bonded downstream-type bonding-enabled
no cable qos permission create

no cable qos permission update

cable qos permission modems

cable multicast group-qos default scn def_sclass aggregate

cable multicast group-qos 1 scn MQoS aggregate

cable multicast group-qos 5 scn MQoS aggregate

!

cable multicast qos group 1 priority 1  

 session-range 225.0.0.0 255.0.0.0

 group-qos 1

!

cable multicast qos group 5 priority 5  

 session-range 239.1.1.0 255.255.255.0

 session-range 225.1.1.0 255.255.255.0

 group-qos 5
!

cable time-server

cable throttle-ranging

cable diaglog active

cable load-balance group 1 method utilization

cable load-balance group 1 interval 30

cable load-balance group 1 dcc-init-technique 1

cable load-balance group 1 threshold load 10 enforce 18

cable load-balance group 20 method utilization

cable load-balance group 20 interval 30

cable load-balance group 20 dcc-init-technique 1

cable load-balance group 20 threshold load 10 enforce 22

cable load-balance group 20 policy us-groups-across-ds

cable load-balance exclude modem 00e0.6f72.84b0 strict

cable load-balance exclude modem 00e0.6f86.4aaa strict

cable load-balance exclude modem 00e0.6fdc.981c strict

cable load-balance exclude oui 00.21.43

cable load-balance exclude oui 00.1A.C3

cable clock dti
cable wideband auto-reset

!

cable rcc-template 1

  rcp-id 00 10 18 33 81

  receive-module 1 first-center-frequency 609000000

  receive-channel 1 center-frequency 609000000 connected-receive-module 1 primary

  receive-channel 2 center-frequency 615000000 connected-receive-module 1

  receive-channel 3 center-frequency 621000000 connected-receive-module 1

!

cable rcc-template 2

  rcp-id 00 10 18 33 81

  receive-module 1 first-center-frequency 609000000

  receive-channel 1 center-frequency 609000000 connected-receive-module 1 primary

  receive-channel 2 center-frequency 615000000 connected-receive-module 1

  receive-channel 3 center-frequency 627000000 connected-receive-module 1

!

cable rcc-template 3

  rcp-id 00 10 18 33 81

  receive-module 1 first-center-frequency 609000000

  receive-channel 1 center-frequency 609000000 connected-receive-module 1 primary

  receive-channel 2 center-frequency 615000000 connected-receive-module 1

  receive-channel 3 center-frequency 633000000 connected-receive-module 1

!

cable rcc-template 4

  rcp-id 00 10 00 00 03

  receive-module 1 first-center-frequency 639000000

  receive-channel 1 center-frequency 639000000 connected-receive-module 1 primary

  receive-channel 2 center-frequency 645000000 connected-receive-module 1

  receive-channel 3 center-frequency 651000000 connected-receive-module 1

  receive-channel 4 center-frequency 657000000 connected-receive-module 1
!

ip subnet-zero

no ip domain lookup

ip multicast-routing 

ip dhcp relay information option

no ip dhcp use vrf connected

ip dhcp excluded-address 10.10.152.1

ip dhcp excluded-address 61.18.187.1

!

ip dhcp pool management

   network 192.168.9.0 255.255.255.0

   default-router 192.168.9.2 

!

ip dhcp pool pc

   network 61.18.187.0 255.255.255.0

   next-server 61.18.187.1 

   default-router 61.18.187.1 

   domain-name cisco.com

   dns-server 61.18.187.1 

   lease 7 0 10

!

ip dhcp pool cm

   network 10.10.152.0 255.255.248.0

   bootfile basic30.cfg

   next-server 10.10.152.1 

   option 2 hex ffff.b9b0

   default-router 10.10.152.1 

   option 7 ip 10.10.152.1 

   option 4 ip 10.10.152.1 

   lease 5

!

!

!

!

!

!

!

redundancy

 mode sso

!

ip access-list extended mqosacl

 permit ip any 225.0.0.0 0.255.255.255

!

controller Modular-Cable 1/0/0

 ip-address 192.168.200.45

 modular-host subslot 6/1

 rf-channel 0 cable downstream channel-id 24

 rf-channel 0 frequency 609000000 annex B modulation 256qam interleave 32

 rf-channel 0 ip-address 192.168.200.40 mac-address 0050.4b11.3930 depi-remote-id 37001

 rf-channel 1 cable downstream channel-id 25

 rf-channel 1 frequency 615000000 annex B modulation 256qam interleave 32

 rf-channel 1 ip-address 192.168.200.40 mac-address 0050.4b11.3930 depi-remote-id 37002

 rf-channel 2 cable downstream channel-id 26

 rf-channel 2 frequency 621000000 annex B modulation 256qam interleave 32

 rf-channel 2 ip-address 192.168.200.40 mac-address 0050.4b11.3930 depi-remote-id 37003

 rf-channel 3 cable downstream channel-id 27

 rf-channel 3 frequency 627000000 annex B modulation 256qam interleave 32

 rf-channel 3 ip-address 192.168.200.40 mac-address 0050.4b11.3930 depi-remote-id 37004

 rf-channel 4 cable downstream channel-id 28

 rf-channel 4 frequency 633000000 annex B modulation 256qam interleave 32

 rf-channel 4 ip-address 192.168.200.40 mac-address 0050.4b11.3930 depi-remote-id 37005

 rf-channel 5 cable downstream channel-id 29

 rf-channel 5 frequency 639000000 annex B modulation 256qam interleave 32

 rf-channel 5 ip-address 192.168.200.40 mac-address 0050.4b11.3930 depi-remote-id 37006

 rf-channel 6 cable downstream channel-id 30

 rf-channel 6 frequency 645000000 annex B modulation 256qam interleave 32

 rf-channel 6 ip-address 192.168.200.40 mac-address 0050.4b11.3930 depi-remote-id 37007

 rf-channel 7 cable downstream channel-id 31

 rf-channel 7 frequency 651000000 annex B modulation 256qam interleave 32

 rf-channel 7 ip-address 192.168.200.40 mac-address 0050.4b11.3930 depi-remote-id 37008

 rf-channel 8 cable downstream channel-id 32

 rf-channel 8 frequency 657000000 annex B modulation 256qam interleave 32

 rf-channel 8 ip-address 192.168.200.40 mac-address 0050.4b11.3930 depi-remote-id 37009

 rf-channel 9 cable downstream channel-id 33

 rf-channel 10 cable downstream channel-id 34

 rf-channel 11 cable downstream channel-id 35

 rf-channel 12 cable downstream channel-id 36

 rf-channel 13 cable downstream channel-id 37

 rf-channel 14 cable downstream channel-id 38

 rf-channel 15 cable downstream channel-id 39

 rf-channel 16 cable downstream channel-id 40

 rf-channel 17 cable downstream channel-id 41

 rf-channel 18 cable downstream channel-id 42

 rf-channel 19 cable downstream channel-id 43

 rf-channel 20 cable downstream channel-id 44

 rf-channel 21 cable downstream channel-id 45

 rf-channel 22 cable downstream channel-id 46

 rf-channel 23 cable downstream channel-id 47

!

! 

!

!

!

!

interface Loopback0

 ip address 10.1.1.1 255.255.255.255

!

interface FastEthernet0/0/0

 ip address 192.168.9.2 255.255.255.0

 load-interval 30

 speed auto

 duplex auto

!

interface Wideband-Cable1/0/0:0

 load-interval 30

 cable bundle 1

 cable bonding-group-id 1

 cable rf-channel 0 bandwidth-percent 95

 cable rf-channel 1

!

interface Wideband-Cable1/0/0:1

 load-interval 30

 cable bundle 1

 cable igmp static-group 239.1.1.1

 cable igmp static-group 239.1.1.7

 cable igmp static-group 239.1.1.8

 cable bonding-group-id 2 secondary

 cable rf-channel 2

!

interface Wideband-Cable1/0/0:2

 load-interval 30

 cable bundle 1

 cable igmp static-group 239.1.1.3

 cable igmp static-group 239.1.1.4

 cable bonding-group-id 3 secondary

 cable rf-channel 3

!

interface Wideband-Cable1/0/0:3

 load-interval 30

 cable bundle 1

 cable igmp static-group 239.1.1.5

 cable igmp static-group 239.1.1.6

 cable bonding-group-id 4 secondary

 cable rf-channel 4
!

interface Wideband-Cable1/0/0:4

 load-interval 30

 cable bundle 1

 cable bonding-group-id 5

 cable rf-channel 5 bandwidth-percent 95

 cable rf-channel 6

 cable rf-channel 7

 cable rf-channel 8

!

interface Wideband-Cable1/0/0:5

 cable bonding-group-id 6

!

interface Wideband-Cable1/0/0:6

 cable bonding-group-id 7

!

interface Wideband-Cable1/0/0:7

 cable bonding-group-id 8

!

interface Wideband-Cable1/0/0:8

 cable bonding-group-id 9

!

interface Wideband-Cable1/0/0:9

 cable bonding-group-id 33

!

interface Wideband-Cable1/0/0:10

 cable bonding-group-id 34

!

interface Wideband-Cable1/0/0:11

 cable bonding-group-id 35

!

interface Modular-Cable1/0/0:0

 load-interval 30

 cable bundle 1

 cable rf-bandwidth-percent 1

 cable cm-status enable 1-2 4-5

!

interface Modular-Cable1/0/0:1

 load-interval 30

 cable bundle 1

 no cable rf-bandwidth-percent

 cable cm-status enable 1-2 4-5

!

interface Modular-Cable1/0/0:2

 load-interval 30

 cable bundle 1

 no cable rf-bandwidth-percent

 cable cm-status enable 1-2 4-5

!

interface Modular-Cable1/0/0:3

 load-interval 30

 cable bundle 1

 no cable rf-bandwidth-percent

 cable cm-status enable 1-2 4-5

!

interface Modular-Cable1/0/0:4

 load-interval 30

 cable bundle 1

 no cable rf-bandwidth-percent

 cable cm-status enable 1-2 4-5

!

interface Modular-Cable1/0/0:5

 load-interval 30

 cable bundle 1

 cable rf-bandwidth-percent 1

 cable cm-status enable 1-2 4-5

!

interface Modular-Cable1/0/0:6

 cable bundle 1

 no cable rf-bandwidth-percent

 no cable cm-status enable 1-2 4-5

!

interface Modular-Cable1/0/0:7

 load-interval 30

 cable bundle 1

 no cable rf-bandwidth-percent

 cable cm-status enable 1-2 4-5

!

interface Modular-Cable1/0/0:8

 load-interval 30

 cable bundle 1

 no cable rf-bandwidth-percent

 cable cm-status enable 1-2 4-5

!

interface GigabitEthernet4/0/0

 ip address 10.254.9.52 255.255.255.192

 ip pim sparse-mode

 ip ospf priority 0

 load-interval 30

 negotiation auto

!

interface GigabitEthernet4/1/0

 no ip address

 negotiation auto

!

interface Cable6/1/0

 load-interval 30

 downstream local upstream 0

 downstream Modular-Cable 1/0/0 rf-channel 0-8

 cable rcc-template 3

 cable rcc-template 2

 cable rcc-template 1

 cable rcc-template 4
 cable cm-status enable 1-2 4-5

 no cable packet-cache

 cable default-phy-burst 0

 cable map-advance dynamic 600 600

 cable bundle 1

 cable downstream channel-id 148

 cable downstream annex B

 cable downstream modulation 256qam

 cable downstream interleave-depth 32

 cable downstream frequency 657000000

 cable downstream rf-shutdown

 cable upstream max-ports 4

 cable upstream 0 connector 0

 cable upstream 0 frequency 20200000

 cable upstream 0 docsis-mode atdma

 cable upstream 0 channel-width 6400000 6400000

 cable upstream 0 minislot-size 2

 cable upstream 0 power-level 5

 cable upstream 0 fragment-force 2000

 cable upstream 0 range-backoff 3 6

 cable upstream 0 modulation-profile 221

 no cable upstream 0 shutdown

 cable upstream 1 connector 1

 cable upstream 1 frequency 26600000

 cable upstream 1 docsis-mode atdma

 cable upstream 1 channel-width 6400000 6400000

 cable upstream 1 minislot-size 1

 cable upstream 1 power-level 5

 cable upstream 1 range-backoff 3 6

 cable upstream 1 modulation-profile 221

 cable upstream 1 load-balance group 1

 cable upstream 1 shutdown

 cable upstream 2 connector 2

 cable upstream 2 docsis-mode tdma

 cable upstream 2 channel-width 1600000 1600000

 cable upstream 2 minislot-size 4

 cable upstream 2 range-backoff 3 6

 cable upstream 2 modulation-profile 21

 cable upstream 2 shutdown

 cable upstream 3 connector 3

 cable upstream 3 docsis-mode tdma

 cable upstream 3 channel-width 1600000 1600000

 cable upstream 3 minislot-size 4

 cable upstream 3 range-backoff 3 6

 cable upstream 3 modulation-profile 21

 cable upstream 3 shutdown

!

interface Cable6/1/1

 cable cm-status enable 1-2 4-5

 no cable packet-cache

 cable downstream channel-id 149

 cable downstream annex B

 cable downstream modulation 64qam

 cable downstream interleave-depth 32

 cable downstream rf-shutdown

 cable upstream max-ports 4

 cable upstream 0 connector 4

 cable upstream 0 docsis-mode tdma

 cable upstream 0 channel-width 1600000 1600000

 cable upstream 0 minislot-size 4

 cable upstream 0 range-backoff 3 6

 cable upstream 0 modulation-profile 21

 cable upstream 0 shutdown

 cable upstream 1 connector 5

 cable upstream 1 docsis-mode tdma

 cable upstream 1 channel-width 1600000 1600000

 cable upstream 1 minislot-size 4

 cable upstream 1 range-backoff 3 6

 cable upstream 1 modulation-profile 21

 cable upstream 1 shutdown

 cable upstream 2 connector 6

 cable upstream 2 docsis-mode tdma

 cable upstream 2 channel-width 1600000 1600000

 cable upstream 2 minislot-size 4

 cable upstream 2 range-backoff 3 6

 cable upstream 2 modulation-profile 21

 cable upstream 2 shutdown

 cable upstream 3 connector 7

 cable upstream 3 docsis-mode tdma

 cable upstream 3 channel-width 1600000 1600000

 cable upstream 3 minislot-size 4

 cable upstream 3 range-backoff 3 6

 cable upstream 3 modulation-profile 21

 cable upstream 3 shutdown

!

interface Cable6/1/2

 cable cm-status enable 1-2 4-5

 no cable packet-cache

 cable downstream channel-id 150

 cable downstream annex B

 cable downstream modulation 64qam

 cable downstream interleave-depth 32

 cable downstream rf-shutdown

 cable upstream max-ports 4

 cable upstream 0 connector 8

 cable upstream 0 docsis-mode tdma

 cable upstream 0 channel-width 1600000 1600000

 cable upstream 0 minislot-size 4

 cable upstream 0 range-backoff 3 6

 cable upstream 0 modulation-profile 21

 cable upstream 0 shutdown

 cable upstream 1 connector 9

 cable upstream 1 docsis-mode tdma

 cable upstream 1 channel-width 1600000 1600000

 cable upstream 1 minislot-size 4

 cable upstream 1 range-backoff 3 6

 cable upstream 1 modulation-profile 21

 cable upstream 1 shutdown

 cable upstream 2 connector 10

 cable upstream 2 docsis-mode tdma

 cable upstream 2 channel-width 1600000 1600000

 cable upstream 2 minislot-size 4

 cable upstream 2 range-backoff 3 6

 cable upstream 2 modulation-profile 21

 cable upstream 2 shutdown

 cable upstream 3 connector 11

 cable upstream 3 docsis-mode tdma

 cable upstream 3 channel-width 1600000 1600000

 cable upstream 3 minislot-size 4

 cable upstream 3 range-backoff 3 6

 cable upstream 3 modulation-profile 21

 cable upstream 3 shutdown

!

interface Cable6/1/3

 cable cm-status enable 1-2 4-5

 no cable packet-cache

 cable downstream channel-id 151

 cable downstream annex B

 cable downstream modulation 64qam

 cable downstream interleave-depth 32

 cable downstream rf-shutdown

 cable upstream max-ports 4

 cable upstream 0 connector 12

 cable upstream 0 docsis-mode tdma

 cable upstream 0 channel-width 1600000 1600000

 cable upstream 0 minislot-size 4

 cable upstream 0 range-backoff 3 6

 cable upstream 0 modulation-profile 21

 cable upstream 0 shutdown

 cable upstream 1 connector 13

 cable upstream 1 docsis-mode tdma

 cable upstream 1 channel-width 1600000 1600000

 cable upstream 1 minislot-size 4

 cable upstream 1 range-backoff 3 6

 cable upstream 1 modulation-profile 21

 cable upstream 1 shutdown

 cable upstream 2 connector 14

 cable upstream 2 docsis-mode tdma

 cable upstream 2 channel-width 1600000 1600000

 cable upstream 2 minislot-size 4

 cable upstream 2 range-backoff 3 6

 cable upstream 2 modulation-profile 21

 cable upstream 2 shutdown

 cable upstream 3 connector 15

 cable upstream 3 docsis-mode tdma

 cable upstream 3 channel-width 1600000 1600000

 cable upstream 3 minislot-size 4

 cable upstream 3 range-backoff 3 6

 cable upstream 3 modulation-profile 21

 cable upstream 3 shutdown

!

interface Cable6/1/4

 cable cm-status enable 1-2 4-5

 no cable packet-cache

 cable downstream channel-id 152

 cable downstream annex B

 cable downstream modulation 64qam

 cable downstream interleave-depth 32

 cable downstream rf-shutdown

 cable upstream max-ports 4

 cable upstream 0 connector 16

 cable upstream 0 docsis-mode tdma

 cable upstream 0 channel-width 1600000 1600000

 cable upstream 0 minislot-size 4

 cable upstream 0 range-backoff 3 6

 cable upstream 0 modulation-profile 21

 cable upstream 0 shutdown

 cable upstream 1 connector 17

 cable upstream 1 docsis-mode tdma

 cable upstream 1 channel-width 1600000 1600000

 cable upstream 1 minislot-size 4

 cable upstream 1 range-backoff 3 6

 cable upstream 1 modulation-profile 21

 cable upstream 1 shutdown

 cable upstream 2 connector 18

 cable upstream 2 docsis-mode tdma

 cable upstream 2 channel-width 1600000 1600000

 cable upstream 2 minislot-size 4

 cable upstream 2 range-backoff 3 6

 cable upstream 2 modulation-profile 21

 cable upstream 2 shutdown

 cable upstream 3 connector 19

 cable upstream 3 docsis-mode tdma

 cable upstream 3 channel-width 1600000 1600000

 cable upstream 3 minislot-size 4

 cable upstream 3 range-backoff 3 6

 cable upstream 3 modulation-profile 21

 cable upstream 3 shutdown

!

interface Bundle1

 ip address 10.8.208.1 255.255.252.0 secondary

 ip address 61.18.187.1 255.255.255.0 secondary

 ip address 10.10.152.1 255.255.248.0

 ip pim sparse-mode

 ip ospf priority 0

 ip igmp static-group 239.1.1.1

 ip igmp static-group 239.1.1.2

 ip igmp static-group 239.1.1.3

 ip igmp static-group 239.1.1.4

 ip igmp static-group 239.1.1.5

 ip igmp static-group 239.1.1.6

 ip igmp static-group 239.1.1.7

 ip igmp static-group 239.1.1.8
 ip igmp version 3

 load-interval 30

 cable multicast-qos group 5
 cable arp filter request-send 3 2

 cable arp filter reply-accept 3 2

 no cable ip-multicast-echo
 cable dhcp-giaddr policy

 cable helper-address 192.168.128.71

!

router ospf 1

 log-adjacency-changes

 area 9 stub no-summary

 network 10.8.208.0 0.0.3.255 area 9

 network 10.10.152.0 0.0.7.255 area 9

 network 10.249.11.48 0.0.0.15 area 9

 network 10.254.9.0 0.0.0.63 area 9

 network 61.18.187.0 0.0.0.255 area 9

!

ip classless

ip route 0.0.0.0 0.0.0.0 10.254.9.62 220

!

!

no ip http server

no ip http secure-server

ip pim rp-address 10.1.1.1

!

logging cmts cr10k log-level errors

cpd cr-id 1

nls resp-timeout 1

cdp run

!

tftp-server disk0:uBR_CM.cfg

tftp-server disk0:MaxThroughputCfgFile.bin

tftp-server disk0:basic30.cfg

snmp-server community public RO

snmp-server manager

snmp ifmib ifindex persist

!

!

control-plane

!

alias exec scm show cable modem

alias exec ccm clear cable modem all reset

alias exec ru show run

alias exec scla show cable load all

!

line con 0

line aux 0

line vty 0 4

 exec-timeout 0 0

 no login

line vty 5 15

 no login

!

!

cable fiber-node 1

  downstream Modular-Cable 1/0/0 rf-channel 0-8

  upstream Cable 6/1 connector 0

!

End

7. APPENDIX 3 - UBR10012 configuration sample 2 
The modem is tuned to two 4 channel bonding groups. 
RCC templates are configured for the modem to simultaneously tune to multiple BGs. Multicast QoS with service class attributes is configured to steer multicast traffic to different bonding groups.

RCC Template Configuration:

The RCC templates will be different for different modems on the UBR10K.
The RCC templates mentioned below have the same frequencies for the secondary BG (Wi1/0/0:2) which is RF-spanned across different SGs. In this case, the narrowcast BG (the first four frequencies) are unique (Wi1/0/0:0 and Wi1/0/0:1) and the modems can be tuned to any one of those.

Sample example for RCC template for a DPC3010 modem:

cable rcc-template 1
  rcp-id 00 10 18 80 61

  receive-module 1 first-center-frequency 453000000

  receive-module 2 first-center-frequency 501000000

  receive-channel 1 center-frequency 453000000 connected-receive-module 1 primary

  receive-channel 2 center-frequency 459000000 connected-receive-module 1

  receive-channel 3 center-frequency 465000000 connected-receive-module 1

  receive-channel 4 center-frequency 471000000 connected-receive-module 1

  receive-channel 5 center-frequency 501000000 connected-receive-module 2

  receive-channel 6 center-frequency 507000000 connected-receive-module 2

  receive-channel 7 center-frequency 513000000 connected-receive-module 2

  receive-channel 8 center-frequency 519000000 connected-receive-module 2

cable rcc-template 2

  rcp-id 00 10 18 80 61

  receive-module 1 first-center-frequency 477000000

  receive-module 2 first-center-frequency 501000000

  receive-channel 1 center-frequency 477000000 connected-receive-module 1 primary

  receive-channel 2 center-frequency 483000000 connected-receive-module 1

  receive-channel 3 center-frequency 489000000 connected-receive-module 1

  receive-channel 4 center-frequency 495000000 connected-receive-module 1

  receive-channel 5 center-frequency 501000000 connected-receive-module 2

  receive-channel 6 center-frequency 507000000 connected-receive-module 2

  receive-channel 7 center-frequency 513000000 connected-receive-module 2

  receive-channel 8 center-frequency 519000000 connected-receive-module 2
MQoS and service class Configuration:

It is assumed here that the popular content channels will fall in the session range 235.2.0.0 255.255.0.0 and the long tail content will be forwarded on the primary BG.
cable service class 1 name mcast_sclass

cable service class 1 downstream

cable service class 1 max-rate 0

cable service class 1 req-attr-mask FF

cable multicast group-qos default scn mcast_sclass aggregate

cable multicast group-qos 1 scn mcast_sclass single

!

cable multicast qos group 1 priority 1  

 session-range 235.2.0.0 255.255.0.0

 group-qos 1

!
Bonding group configuration:

interface Wideband-Cable1/1/0:0

 load-interval 30

 cable bundle 2

 cable bonding-group-id 33

 cable dynamic-bw-sharing

 cable rf-channel 0 bandwidth-percent 95

 cable rf-channel 1

 cable rf-channel 2

 cable rf-channel 3

!

interface Wideband-Cable1/1/0:0

 load-interval 30

 cable bundle 2

 cable bonding-group-id 33

 cable dynamic-bw-sharing

 cable rf-channel 4 bandwidth-percent 95

 cable rf-channel 5

 cable rf-channel 6

 cable rf-channel 7

!

interface Wideband-Cable1/1/0:1

 load-interval 30

 cable bundle 2

 cable igmp static-group 235.2.10.1

 cable igmp static-group 235.2.10.2

 cable igmp static-group 235.2.10.3

 cable igmp static-group 235.2.10.4

 cable igmp static-group 235.2.10.5

 cable igmp static-group 235.2.10.6

 cable igmp static-group 235.2.10.7

 cable igmp static-group 235.2.10.8

 cable igmp static-group 235.2.10.9

 cable igmp static-group 235.2.10.10

 cable igmp static-group 235.2.10.11

 cable igmp static-group 235.2.10.12

 cable igmp static-group 235.2.10.13

 cable igmp static-group 235.2.10.14

 cable igmp static-group 235.2.10.15

 cable bonding-group-id 34

 cable rf-channel 8

 cable rf-channel 9

 cable rf-channel 10

 cable rf-channel 11

 cable downstream attribute-mask 800000FF

Cable Bundle Configuration:

interface Bundle2

 ip address 192.168.50.1 255.255.255.0 secondary

 ip address 172.20.1.1 255.255.255.0

 ip pim sparse-mode

 ip igmp static-group 235.2.10.1

 ip igmp static-group 235.2.10.2

 ip igmp static-group 235.2.10.3

 ip igmp static-group 235.2.10.4

 ip igmp static-group 235.2.10.5

 ip igmp static-group 235.2.10.6

 ip igmp static-group 235.2.10.7

 ip igmp static-group 235.2.10.8

 ip igmp static-group 235.2.10.9

 ip igmp static-group 235.2.10.10

 ip igmp static-group 235.2.10.11

 ip igmp static-group 235.2.10.12

 ip igmp static-group 235.2.10.13

 ip igmp static-group 235.2.10.14

 ip igmp static-group 235.2.10.15

 ip igmp version 3

 cable multicast-qos group 1
 cable multicast-qos group 2
 cable arp filter request-send 3 2

 cable arp filter reply-accept 3 2

 no cable ip-multicast-echo

 cable dhcp-giaddr policy

 cable helper-address 172.22.71.250

!
Ensuring that different modems tune to different narrowcast BGs:

cable load-balance group 1 threshold load 1

cable load-balance group 1 threshold load minimum 1

Also under the LC host:

interface Cable7/0/0

cable load-balance group 1






Note Up to 8 upstream to a single MAC domain, Up to 20 downstream to a single MAC domain


Note If operating ambient temperature is kept below 40 degree Celcius


Note Assuming 5 upstream modem is available. If not then bonding groups of less than 5 upstream can be configured to use all upstreams.





Introducing the MC20x20V-20D15-Jun-10
Page 69 of 53

_1324114616.vsd
text


Frequency


MC20x20V-20D
LINE CARD


us


us


us


us


 MAC 
Domain


PRIMARY


PRIMARY


PRIMARY


SERVICE 
AREA 1


PRIMARY


usf1


usf0


dsf4


dsf5


dsf6


dsf7


usf2


usf3


us


us


us


us


us


us


us


us


 MAC 
Domain


us


us


us


us


us


PRIMARY


PRIMARY


PRIMARY


PRIMARY


PRIMARY


 MAC 
Domain


us


SECONDARY


SECONDARY


SECONDARY


SECONDARY


SERVICE 
AREA 3


PRIMARY


PRIMARY


SERVICE 
AREA 4


PRIMARY


PRIMARY


PRIMARY


PRIMARY


PRIMARY


 MAC 
Domain


 MAC 
Domain


4 CHANNEL BONDING


SHARED 8 CHANNEL BONDING


SHARED 4 CHANNEL BONDED IPTV


us


PRIMARY


DOCSIS 2.0 SUPPORT


us


SERVICE 
AREA 2


dsf1


dsf2


dsf3


dsf4



_1324190465.vsd
text


Frequency


MC20x20V-20D
LINE CARD


us


us


us


us


 MAC 
Domain


SERVICE 
AREA 1


PRIMARY


PRIMARY


PRIMARY


PRIMARY


usf1


usf0


dsf1


dsf2


dsf3


dsf4


usf2


usf3


us


us


us


us


us


us


us


us


 MAC 
Domain


us


us


us


us


PRIMARY


 MAC 
Domain


us


SERVICE 
AREA 3


SERVICE 
AREA 4


us


us


 MAC 
Domain


 MAC 
Domain


SHARED 8 CHANNEL BONDING


us


SERVICE 
AREA 5


PRIMARY


DOCSIS 2.0 SUPPORT


SERVICE 
AREA 2


dsf5


dsf6


dsf7


dsf8



_1328602660.vsd
text


us


MC20x20V-20D
LINE CARD


us


us


us


us


us


us


us


SERVICE 
AREA 2


PRIMARY


PRIMARY


PRIMARY


PRIMARY


 MAC 
Domain


PRIMARY


PRIMARY


PRIMARY


SERVICE 
AREA 1


PRIMARY


usf1


usf0


dsf1


dsf2


dsf3


dsf4


usf2


usf3


 MAC 
Domain


4 CHANNEL BONDING


PRIMARY



_1341811208.vsd
text


Frequency


MC20x20V-20D
LINE CARD


us


us


us


us


 MAC 
Domain


PRIMARY


PRIMARY


PRIMARY


SERVICE 
AREA 1


PRIMARY


usf1


usf0


dsf4


dsf5


dsf6


dsf7


usf2


usf3


us


us


us


us


us


us


us


us


 MAC 
Domain


us


us


us


us


us


PRIMARY


PRIMARY


PRIMARY


PRIMARY


PRIMARY


 MAC 
Domain


us


SECONDARY


SECONDARY


SECONDARY


SECONDARY


SERVICE 
AREA 3


PRIMARY


PRIMARY


SERVICE 
AREA 4


PRIMARY


PRIMARY


PRIMARY


PRIMARY


PRIMARY


 MAC 
Domain


 MAC 
Domain


4 CHANNEL BONDING


SHARED 8 CHANNEL BONDING


SHARED 4 CHANNEL BONDED IPTV


us


PRIMARY


DOCSIS 2.0 SUPPORT


us


SERVICE 
AREA 2


dsf1


dsf2


dsf3


dsf4


TX0


TX1


TX2


TX3


TX4


MC20x20V-20D


Service Area



_1324890239.vsd
text


dsf5


us


MC20x20V-20D
LINE CARD


us


us


us


us


us


us


us


8 CHANNEL BONDING


PRIMARY


PRIMARY


PRIMARY


PRIMARY


us


us


us


us


SERVICE 
AREA 2


PRIMARY


PRIMARY


PRIMARY


PRIMARY


PRIMARY


PRIMARY


SERVICE 
AREA 1


PRIMARY


PRIMARY


 MAC 
Domain


us


us


us


us


SERVICE 
AREA 4


usf1


usf0


dsf1


dsf2


dsf3


dsf4


usf2


usf3


PRIMARY


PRIMARY


PRIMARY


PRIMARY


 MAC 
Domain


us


us


us


us


PRIMARY


PRIMARY


PRIMARY


PRIMARY


 MAC 
Domain


dsf6


dsf7


dsf8


 MAC 
Domain


4 CHANNEL BONDING


PRIMARY


DOCSIS 2.0 SUPPORT



_1324115339.vsd
text


Frequency


MC20x20V-20D
LINE CARD


us


us


us


us


 MAC 
Domain


PRIMARY


PRIMARY


PRIMARY


SERVICE 
AREA 1


PRIMARY


usf1


usf0


dsf1


dsf2


dsf3


dsf4


usf2


usf3


us


us


us


us


us


us


us


us


 MAC 
Domain


us


PRIMARY


PRIMARY


PRIMARY


PRIMARY


PRIMARY


 MAC 
Domain


SECONDARY


SECONDARY


SECONDARY


SECONDARY


SERVICE 
AREA 3


PRIMARY


PRIMARY


SECONDARY


SECONDARY


us


PRIMARY


SECONDARY


SECONDARY


dsf5


dsf6


dsf7


dsf8


 MAC 
Domain


 MAC 
Domain


4 CHANNEL BONDING


SHARED 8 CHANNEL BONDING


dsf9


dsf10


dsf11


dsf12


SHARED 8 CHANNEL BONDED IPTV


us


PRIMARY


DOCSIS 2.0 SUPPORT


us


SERVICE 
AREA 2


us


us


us


us



_1324116326.vsd
text


dsf5


us


MC20x20V-20D
LINE CARD


us


us


us


us


us


us


us


8 CHANNEL BONDING


PRIMARY


PRIMARY


PRIMARY


PRIMARY


us


us


us


us


SERVICE 
AREA 2


PRIMARY


PRIMARY


PRIMARY


PRIMARY


PRIMARY


PRIMARY


SERVICE 
AREA 1


PRIMARY


PRIMARY


 MAC 
Domain


us


us


us


us


SERVICE 
AREA 3


usf1


usf0


dsf1


dsf2


dsf3


dsf4


usf2


usf3


PRIMARY


PRIMARY


PRIMARY


PRIMARY


 MAC 
Domain


us


us


SECONDARY


SECONDARY


SECONDARY


SECONDARY


us


us


 MAC 
Domain


dsf6


dsf7


dsf8


 MAC 
Domain


4 CHANNEL BONDING


PRIMARY


DOCSIS 2.0 SUPPORT



_1324115234.vsd
text


Frequency


MC20x20V-20D
LINE CARD


us


us


us


us


 MAC 
Domain


PRIMARY


PRIMARY


PRIMARY


SERVICE 
AREA 1


PRIMARY


usf1


usf0


dsf4


dsf5


dsf6


dsf7


usf2


usf3


us


us


us


us


us


us


us


us


 MAC 
Domain


us


us


us


us


us


PRIMARY


PRIMARY


PRIMARY


PRIMARY


PRIMARY


 MAC 
Domain


us


SECONDARY


SECONDARY


SECONDARY


SECONDARY


SERVICE 
AREA 3


PRIMARY


PRIMARY


SERVICE 
AREA 4


PRIMARY


PRIMARY


PRIMARY


PRIMARY


PRIMARY


 MAC 
Domain


 MAC 
Domain


4 CHANNEL BONDING


SHARED 8 CHANNEL BONDING


SHARED 4 CHANNEL BONDED IPTV


us


PRIMARY


DOCSIS 2.0 SUPPORT


us


SERVICE 
AREA 2


dsf1


dsf2


dsf3


dsf4



_1296372626/239.1.1.2.bat
vlc "C:\Video\2- jane_DMS_4_training.mpg" --sout=#duplicate{dst=std{access=udp{ttl=10},mux=ts,dst=239.1.1.2:1234}}


_1319538355.vsd
text


us


MC20x20V-20D
LINE CARD


us


us


us


us


us


us


us


SERVICE 
AREA 2


PRIMARY


PRIMARY


PRIMARY


PRIMARY


 MAC 
Domain


us


us


us


us


SERVICE 
AREA 3


PRIMARY


PRIMARY


PRIMARY


PRIMARY


PRIMARY


PRIMARY


SERVICE 
AREA 1


PRIMARY


PRIMARY


 MAC 
Domain


us


us


us


us


SERVICE 
AREA 4


usf1


usf0


dsf1


dsf2


dsf3


dsf4


usf2


usf3


PRIMARY


PRIMARY


PRIMARY


PRIMARY


 MAC 
Domain


us


us


us


us


SERVICE 
AREA 5


PRIMARY


PRIMARY


PRIMARY


PRIMARY


 MAC 
Domain


 MAC 
Domain


4 CHANNEL BONDING


PRIMARY


DOCSIS 2.0 SUPPORT



_1296372623/239.1.1.1.bat
vlc "C:\Video\1- 2370_teleprez_business_STEREO.mpg" --sout=#duplicate{dst=std{access=udp{ttl=10},mux=ts,dst=239.1.1.1:1234}}


